056:131 Manufacturing Systems			NAME ___________________

FINAL EXAM   Date: April 29, 2004

------------------------------

  - Describe how the equation for design efficiency in the Boothroyd/Dewhurst approach is derived.

  - A redesign effort for a product is expected to take 1000 hours at a cost of $85 per hour. The resultant redesign is expected to require an extra $1500 in manufacturing tooling and fixtures. The redesign is expected eliminate $25 in parts per unit, and require 1 hour less per unit to assemble with an assembly cost of $30 per hour. The parts in the redesigned product are expected to require a total additional manufacturing cost of $10. Calculate the minimum amount to be produced at which the redesign is economically viable.

  - Describe the following terms in relation to lean manufacturing: Kaizen, poka-yoke. Give an example of each.

  - What do the initials DMAIC stand for in relation to Six Sigma? Briefly describe each.

  - What is meant by “value” in Value Stream Mapping? What is meant by the “takt” time in Value Stream Mapping?

  - Produce an exponentially smoothed forecast for period 4 when the initial estimate of demand is 100 and actual demands in periods 1 – 3 are 111, 84, 92, 108. Use an exponential smoothing constant of 0.2.

  - A forecast has errors of 0.4, -0.7, -0.4. The forecast error variance (from past data) is 0.64. Test, using a tracking signal, to see if the forecast is unbiased (use 3 s.d. confidence intervals and a smoothing constant of 0.3).