Manufacturing Systems Spring 2004		NAME ____Solution (20 Points)______

Quiz 2 Date: February 5, 2004

☺☺☺☺☺

  - (4) Name the four main types of manufacturing layout

Process

Product

Cellular

Fixed-position

  - (4) Show where the four types are approximately located on a graph, with variety along the horizontal axis and volume on the vertical axis

  - (2) An item that costs $874 has a reorder point of 250, with a mean demand of 20 per day and an expected lead time of 5 days. What is the safety stock level?

SS = r – τD = 250 – 20*5 = 150

  - (5) Give 5 reasons why inventory is kept.

Effective preventative maintenance

Price breaks for purchasing large quantities

Unreliable suppliers

Seasonal demand

Readily available products

  - (3) How is mass production used in the Dell supply chain?

Parts are mass produced by suppliers then shipped to Dell distribution centers where they are assembled to customer specifications then shipped out to the respective customers.  During the assembly process, each person completes a specialized task then sends the product down the line to another individual.

  - (2) Describe an advantage that push systems have over pull systems and an  advantage that pull systems have over push systems

Push systems – higher inventory, products are readily available

Pull systems – lower inventory, made to order products