Manufacturing Systems Spring 2004	                    NAME __SOLUTION__(17 Points)__________

Quiz 4 Date: February 12, 2004

☺☺☺☺☺

  - (3 points) What are three major assumptions/approximations behind the EOQ formula?

- Demand is known, static, continuous, deterministic demand

- Constant, fixed cost to place an order

- Inventory carrying cost based on value of average on-hand inventory

- Instantaneous batch delivery following a deterministic lead time

- All replenishment orders are for the same quantity , and that quantity is not limited

- No shortages (stockouts) are allowed, they can be avoided if orders are placed at the right time

  - (4) An item has an ordering cost of $200, an inventory holding cost of $20 per item per year, a demand of 1,000 per month. What is the EOQ?

EOQ=Q∗2ADh

A = fixed cost to place an order = 200

D = demand rate in units per time = 1000/mo or 12000/yr

h = inventory holding cost per unit-time = 20/unit/yr

Q∗2(200)1200020=489.9⇒490

- (3) It takes 10 minutes to make the first Whataburger. How long will it take to make the 1,000th, assuming a 90% learning rate?

Y(n) = Kn-a	2-a = 0.9  a = 0.152

Y(n) = 10(1000)-0.152 = 3.5 minutes

  - (4) List 4 main steps in preparing a final board using SMT.

Solder Paste Printing

HSP Component Placement

GSM Component Placement

Convection Re-flow

Automated Optical Inspection

Flying Scorpion Fixtureless Probe Test

Waterwash

  - (3) What are three major defects in PCB assemblies?

Tombstone

Solder balls

Wrong parts / missing parts

Wrong orientation