Manufacturing Systems Spring 2004	                    NAME ___Solution_(20 Points)__

Quiz 5  Date: February 26, 2004

☺☺☺☺☺

- SofasRUs is deciding whether to make or buy a new sofa module.  The present worth of the cost of equipment needed to be purchased to produce the sofa module is $250,000 and training to use the equipment is a further $25,000. The variable production cost is $75 per unit. The vendor can supply the sofa module at a price of  $145/unit. What is the breakeven point at which SofasRUs should purchase the item form the vendor?

250 000 + 25 000 + 75x = 145x

275 000/70 = x = 3929

  - List 2 advantages and 2 disadvantages of making to stock compared with making to order.

Adv: Economies of scale, quick product delivery

Disadv: Inventory cost, obsolescence

  - List three main problems with using approaches that include factors that are difficult to quantify.

Difficult to weigh importance factors, needs to be normalized, tedious to calculate, just an estimate, can be biased

  - What are the three main reasons why it is difficult to have a single design of a product that is suitable for all countries?

Cultural differences/languages

Standards (e.g. government regulations)

User preferences/needs (regional/customer)

- A single stage supply chain consists of a retailer and manufacturer. The lead times for orders to reach the manufacturer is 1 week and the delivery time is 1 week. The retailer aims to keep 2 weeks safety stock. If the retailer likes to order enough to cover the demand for the week + the next 2 weeks + the safety stock. Show in the table below how the data for the supply chain will change when the demand changes in period 2 from 200 to 230. Note that the demand occurs after the quantity has been ordered for that week.

| Week | Starting Inventory | On Order | Order Up To Target | Order Quantity | Demand |
| --- | --- | --- | --- | --- | --- |
| 1 | 600 | 200 | 1000 | 200 | 200 |
| 2 | 600 | 200 | 1000 | 200 | 230 |
| 3 | 570 | 200 | 1150 | 380 | 200 |
| 4 | 570 | 380 | 1000 | 50 | 200 |
| 5 | 750 | 50 | 1000 | 200 | 200 |
| 6 | 600 | 200 | 1000 | 200 | 200 |