Manufacturing Systems Spring 2004	                    NAME ___Solution__(20 Points)___

Quiz 6  Date: March 4, 2004

☺☺☺☺☺

  - (3) Give 3 advantages of using U shaped manufacturing cells over a long production line.

Help out other workers

Shared resources and tools readily available for all

Better communication, workers become a community

Input/output next to each other

  - (3) Give 3 environmental requirements for Kanban Systems

Demand is approximately constant over a planning period

Small setup times

Available, flexible capacity

Disciplined workforce

- (7) Describe how a Kanban system can be used to control material movement between the last 2 workstations in a 4 workstations line. Assume that 2 Kanban squares are used, that the workstations are relatively close together, a single product is manufactured and a single-Kanban system is used.

Workstation 3 makes enough products for workstation 4’s input buffer.  As workstation 4 goes through the product, a large Kanban square is revealed to signal to workstation 3 that 4 is in need of product.  In other words, when 4 needs more product, the large Kanban square is revealed as the signal for 3 to continue/resume production.

  - (3) Calculate the minimum number of Kanbans required to avoid shortages in a workstation loop that has a demand rate of 200 per day, a lead time of 0.25 days, and a container size that holds 4 items. Assume a zero safety factor.

D=200 units/day

T=0.25 days			K = [(200)*(0.25)]/4 = 12.5  13 Kanbans

n=4 units

  - (3) Give 3 reasons why a safety factor may need to be included in 3 a.

Inconsistent demand

Unreliable processes

Long lead time

Unreliable supply chain

Quality issues

Delays