Manufacturing Systems Spring 2004	                    NAME ____Solution (20 points)_____

Date: April 9, 2004

☺☺☺☺☺

- Produce an exponentially smoothed forecast for period 5 when the initial estimate of demand is 70 and actual demands in periods 1 – 4 are 61, 78, 71, 62. Use an exponential smoothing constant of 0.2.

  - What are the advantages and disadvantages of each of the following forecasting methods: Moving Average, Exponential Smoothing, Winter’s Method?

|  | Moving Average | Exponential Smoothing | Winter’s Method |
| --- | --- | --- | --- |
| Advantage | Simple to calculate | Simple to calculate | Good for trends |
| Disadvantage | Doesn’t incorporate trends well | Doesn’t incorporate trends well | Complicated to calculate |

  - For Winters Forecasting method:

    - What is the assumed model of the demand data?

Dt = (B0 + t*G0)st + εt

    - What is the equation used to forecast demand?

Ft,t+τ = (Bt + τ*Gt)*st+τ-N

- A forecast has errors of -2.0, 0.3, 0.2. The forecast error variance (from past data) is 0.49. Test, using a tracking signal, to see if the forecast is unbiased (use 3 s.d. confidence intervals and a smoothing constant of 0.2).

|  | et | Et | Δt | Yt |
| --- | --- | --- | --- | --- |
| 0 |  | 0 | 0.56 |  |
| 1 | -2 | -0.4 | 0.848 | 0.472 |
| 2 | 0.3 | -0.26 | 0.738 | 0.352 |
| 3 | 0.2 | -0.168 | 0.630 | 0.267 |

Range: +/- 0.55*3*(0.2)1/2 = +/- 0.74