Manufacturing Systems					NAME _________________________

Quiz 2 Date: February 10, 2005

☺☺☺☺☺

  - Produce an exponentially smoothed forecast for period 3 when the initial estimate of demand is 70 and actual demands in periods 1 – 2 are 52, 97. Use an exponential smoothing constant of 0.3

S1 = (0.3)(52) + (0.7)(70) = 64.6

S2 = (0.3)(97) + (0.7)(64.6) = 74.32

  - The quarterly sales of Boors Colored Water for the past 2 years are 183, 192, 203, 176, 193, 190, 210,  170. Calculate the initial parameters for Winters Forecasting method.

G0 = (V2 - V1) / 4   = (190.75 – 188.5) / 4 = 0.5625

B0 = V2 + ((N – 1)/ 2)(G0) = 190.75 + ((4 – 1)/2)(0.5625) = 191.59

S0 = D0 / B0 = 170 / 191.59375 = 0.8873

  - A forecast has errors of -3.0, 0.7, -0.1. The forecast error variance (from past data) is 0.64. Test, using a tracking signal, to see if the forecast is unbiased (use 3 s.d. confidence intervals and a smoothing constant of 0.3).

Smoothed Error

E0 = 0

E1 = (0.3)(-3.0) + (0.7)(0) = -0.9

E2 = (0.3)(0.7) + (0.7)(-0.9) = -0.42

E3 = (0.3)(-0.1) + (0.7)(-0.42) = -0.324

Smoothed Absolute Error

Δ0 = (0.8)(√0.64) = 0.64

Δ1 = (0.3)(|-0.3|) +(0.7)(.64)= 1.348

Δ2 = (0.3)(|0.7|) +(0.7)(1.348)= 1.1536

Δ3 = (0.3)(|-0.1|) +(0.7)(1.1536)= 0.83752

Tracking Signals (Et / Δt)

E0 / Δ0 = (0 / 0.64) = 0

E1 / Δ1 = (0.9 / 1.348) = 0.6677

E2 / Δ2 = (0.42 / 1.1536) = 0.3641

E3 / Δ3 = (0.324 / 0.83752) = 0.3869

Acceptable Range

(-0.55) (3) (√0.3) <= Et / Δt <= (0.55) (3) (√0.3)

-0.9037 <= Et / Δt <= 0.9037

Since no tracking signals fall outside of the acceptable range, one can conclude that the forecast is unbiased.

  - If the tracking signal approaches 1, what does it mean?

The forecast errors have the same sign i.e. an upward or downward trend indicating forecast bias

  - What are three major assumptions/approximations behind the EOQ formula.

- Constant, known demand

- Zero replenishment lead time

- No quantity discounts

  - It takes 100 minutes to make the first Widget. How long will it take to make the 500th, assuming a 80% learning rate?

Y(n) = Kn-a

2-a  = 0.80

a = 0.3219

100 = K(1)-0.3219

K = 100

Y(500) = 100 (500)-0.3219  = 13.52 min

  - A material used in batches of 50 has an annual demand of 200 units. The ordering cost is $220, the cost of monitoring the inventory levels is $500 per SKU. The cost of delaying delivery is $125.The cost to hold a part for one year is $7. Should the company stock the item?

h < (A+π)(D)Qns−Cs2/ 2(A)(D) = (220+125)(200)50−5002/ 2(220)(200) = 8.8

Since $7 < $8.8, stock the item