Manufacturing Systems					NAME _________________________

Quiz 3 Date: February 22, 2005

☺☺☺☺☺

- This question concerns aggregating three demands: A, B and C with percentage demand variability and the weights as follows:

|  | A | B | C |
| --- | --- | --- | --- |
| Weight | 0.4 | 0.2 | 0.4 |
| SD (demand variability) | 12 | 13 | 15 |

The covariance matrix V for these three demands is:

| 144 | 80 | -5 |
| --- | --- | --- |
| 80 | 169 | 45 |
| -5 | 45 | 225 |

Calculate the demand variability (standard deviation) of the aggregated demand.

w*V*w’  where w = [ 0.4 0.2 0.4]

Demand Variance = w*V*w’ = 84.2

Demand Variability = √84.2 = 9.18

- A single stage supply chain consists of a retailer and manufacturer. The lead times for orders to reach the manufacturer is 1 week and the delivery time is 1 week. The retailer aims to keep 2 weeks safety stock. If the retailer likes to order enough to cover the demand for the week + the next 2 weeks + the safety stock. Show in the table below how the data for the supply chain will change when the demand changes in period 2 from 100 to 115. Note that the demand occurs after the quantity has been ordered for that week.

| Week | Starting Inventory | On Order | Order Up To Target | Order Quantity | Demand |
| --- | --- | --- | --- | --- | --- |
| 1 | 300 | 100 | 500 | 100 | 100 |
| 2 | 300 | 100 | 500 | 100 | 115 |
| 3 | 285 | 100 | 575 | 190 | 100 |
| 4 | 285 | 190 | 500 | 25 | 100 |
| 5 | 375 | 25 | 500 | 100 | 100 |
| 6 | 300 | 100 | 500 | 100 | 100 |

  - A project justification uses Net Present Qualitative Flow. List three main limitations to this approach.

Difficult to weigh importance factors, needs to be normalized, tedious to calculate, just an estimate, can be biased

  - List 2 main ways in which the demand amplification in question 2 could be reduced.

1. Better forecasting techniques or demand smoothing

2. Better communication/ information sharing

3. Shorter lead times

  - What are two main difficulties with using Mixed Integer Programming Models for typical manufacturing/ supply chain problems?

- Difficult to solve for problems with thousands of products/time consuming

- Integer constraints difficult to solve

- Requires accurate demand measurements for each product