Manufacturing Systems 	Quiz 1 Spring 2006    NAME ___________KEY___________

- The quarterly sales of YetAnotherTruck for the past 1 year (in thousands) are 271, 310, 109, 226. Using Winters method  (with all the exponential smoothing constants 0.3):

  - Calculate the initial parameters F(4), T(4), and c(1-4) using the approaches discussed in class and calculate the forecast for quarter 5 (12)

  - Calculate the updated parameters F, T, and c  for quarters 5 and 6 and the forecast for quarter 6, if demand in quarters 5 and 6  is 258 and 324 respectively. (11)

  - What is the resulting MSD? (11)

| P | A(t) | F(t) | T(t) | C(t) | f(t) |
| --- | --- | --- | --- | --- | --- |
| 1 | 271 |  |  | 1.18 |  |
| 2 | 310 |  |  | 1.35 |  |
| 3 | 109 |  |  | 0.47 |  |
| 4 | 226 | 229 | 0 | 0.98 |  |
| 5 | 258 | 225.7 | -0.981 | 1.17 | 270.9 |
| 6 | 324 | 229.3 | 0.3847 | 1.37 | 303.37 |

MSD= (270.9-258)2 + (303.37-324)2 = 296

2

  - A forecast has errors of -2.0, 0.9, -0.3. The forecast error variance (from past data) is 0.49. Test, using a tracking signal, to see if the forecast is unbiased (use 2 s.d. confidence intervals and a smoothing constant of 0.2).  (25)

| P | et | Et | Δt | ¦ Yt ¦ |
| --- | --- | --- | --- | --- |
| 0 | -- | 0 | 0.56 | -- |
| 1 | -2 | -0.4 | 0.848 | -0.472 |
| 2 | 0.9 | -0.14 | 0.86 | -0.163 |
| 3 | -0.3 | -0.172 | 0.746 | -0.230 |

+/- 0.55 (2) SQRT(0.2) = +/- 0.491  -- Unbiased Forecast

  - If the tracking signal approaches zero, what does it mean? (8)

See Class Notes

- Items are consumed at a constant rate of 100 per month, with an ordering cost of $360 and a holding cost of $1 per item per month. The minimum reorder interval is one month and all order cycles are places on a power-of-2 multiple of months (1, 2 , 4, … months) in order to share truck capacity with other items.

  - What is the least cost reorder interval?  (16)

  - How much does it add to the total cost compared with using the EOQ? (17)

.		.

Q*=268.33	 2.68 Months		Power of 2 : 	|	|	|	2.68 rounds to 2 months

2	2.82	4

½(T’/T* + T*/T’)=1.043   4% cost increase

(continue overleaf if necessary)