Manufacturing Systems 		Quiz 4 Spring 2006    	NAME _________________________

I agree to have my grades posted by this 6 digit number _________  signed _____________________________

Failure to show working will reduce your grade.

- (a) It takes 180 minutes to make the first unit of a new product. How long will it take to make the 800th unit if the learning rate is 0.85?

Y(n) = kn-a	2-a = 0.85	Y(800) = 180*800-0.234 = 37.66

  - How many units will have to be produced before the time of manufacture of a unit is reduced to 60 minutes?

Y(n) = 180*n-0.234 = 109.3

  - Give four reasons for the growth in international trade.

See Class Notes

- (a) Give the four main requirements for an industrial revolution.

See Class Notes

  - List six main characteristics of mass production.

See Class Notes

  - Briefly describe how the conwip approach differs from the kanban approach.

See Class Notes

- Cars  arrive at a toll booth on the Florida Turnpike at a rate of 60 per hour, the average process time is 45 seconds with a standard deviation in the process time of 10 seconds.

  - What is the utilization of the toll booth?

U = ra*te / m	U = 60*0.125 / 1 = 0.75

  - What is the expected number of cars at the booth (including in line)?

WIP =  u / (1-u) = 0.75 / 0.25 = 3 Cars

  - What is the natural coefficient of variation?

Co = σo / to = 10 / 45 = 0.22

  - If the times remain independent, what will be the mean and variance for a group of 20 cars?

Mean = 20 * 45sec		Variance = 20 * 102

- A warehouse in Miami has demand from three sources: Argentina (A) , Belgium (B) and Costa Rica (C)  with demand variability and the weights (proportion of demand) as follows:

|  | A | B | C |
| --- | --- | --- | --- |
| Weight | 0.4 | 0.35 | 0.25 |
| SD (demand variability) | 24 | 20 | 30 |

The covariance between A and B is 530, A and C is 970  and B and C is 1000. Calculate the standard deviation in the resultant demand from these three sources.

w’=0.40.350.25		V=57653097053040010009701000900		w’Vw=714.81   and  s.d. is 26.7