Manufacturing Systems 	 - Show working to receive maximum grade

Mega-Quiz 4 Spring 2007    				NAME _________________________

  - Calculate the minimum number of Kanbans required to avoid shortages in a workstation loop that has a demand rate of 100 per day, a lead time of 0.1 days, and a container size that holds 3 items. Assume a safety factor of 50%.

Ki > 0.1* 100 * (1+.5)/ 3 = 5

  - It takes 90 minutes to make the first unit of a new product. How long will it take to make the 1000th unit if the learning rate is 0.75?

2-a = 0.75   a= -log(0.75)/log(2)  a = 0.415

Y(1000) = 90 * 1000-a

= 5.12 min

  - Using the example in b), how many units will have to be produced before the time of manufacture of a unit is reduced to 70 minutes?

90*n-a = 70         n = 1.832 = 2 units

  - Briefly describe how Pareto analysis can be used in supply chain management.

See Class Notes	. Need to mention calculating cumulative inventory value, classes A, B and C and their respective ratios and work concentrated on Class A

  - What is the Bullwhip effect?

See Class Notes

  - Why is the Bullwhip Effect potential harmful to supply chain participants?

Participants have to respond to much greater variability.

  - Briefly describe three ways in which the Bullwhip Effect can be reduced.

See Class Notes

  - What is contained in the right-hand section of the “House of Quality”?

Comparison with competitor products, customer priorities

  - Briefly describe an example to show how Quality Function Deployment is used.

See class notes

  - In Taguchi robust design, what is a noise factor?

Noise factors are not controllable and can produce an error in the desired response (full grade given for one answer)

  - Describe, with the aid of diagrams, how a CONWIP system differs from a conventional pure-push MRP System.

See text

  - What is the main advantage of CONWIP compared with a conventional pure-push MRP System?

Limits WIP