IE 516 Manufacturing Systems 		Exam I Spring 2009    	NAME _________________________

Failure to show working will reduce your grade.

  - Items are consumed at a constant rate of 200 per month, a holding cost of  $2 per item per month and an ordering cost of $410. The minimum reorder interval is one week and all order cycles are places on a power-of-2 multiple of months (1, 2 , 4, … months) in order to share truck capacity with other items.

    - What is the EOQ?

    - What is the least cost reorder interval?

    - How much does it add to the total cost compared with using the EOQ?

Q* = sqrt(2AD/h) = sqrt((2*410*200)/(2) =286

This is 286/200 months = 1.46

Power of 2 : 	|	|	|	1.46 rounds to 2 months

1	1.41	2

Cost increase  ½(T’/T* + T*/T’)= 0.5(2/1.46  +  1.46/2)  1.049  ie  4.9% cost increase

  - Describe, with the aid of a diagram, “the river of inventory” view of the role of inventory in company operations. How can this view be used to increase efficiency?

See class notes

  - A company has a part with annual demand of 15 that follows a Poisson distribution, with a holding cost of $25 and a backorder cost of $175 per item per year, the lead time is 0.2 year.    What is the optimal reorder point (assume that the demand can be approximated as a normal distribution and that the critical fractile is used in determining r*)?

Mean demand over lead time = 15*.2 = 3, sd = sqrt(3) = 1.732

critical ratio (fratile) = b/(b+h) = 175/(175+25) = .875

Z from normal distribution tables  Φ(z) = 0.875   z =

r* = mean + z x sd = 3 + z x 1.732 =

  - The monthly sales for sales for  a product are 109, 114, 121 for months 1 – 3 respectively. Use exponential smoothing with linear trend and smoothing constants α = 0.2 and β=0.2. Assume that your starting smoothed estimate for month 1 is 108 and your smoothed trend estimate is 0.0. Calculate the forecast for periods 2, 3 and 4, calculated in periods 1, 2 and 3 respectively. Show your working for at least one calculation

Solution

  - If exponential smoothing is used, what weight is given to a demand 2 periods in the past with a smoothing constant of .2?

= .2(1-.2)^2 =  .128

  - The demand for a part follows a Poisson distribution with an average of 3 items per month. What is the probability of the demand being 1 or less in a month?

Prob of 1 or less  = .050 + .149 = 0.199

  - What is the TAKT time?

See class notes

  - A forecast has errors of 0.7, -0.2, 1.1, 0.8, 0.4. The forecast error variance (from past data) is 0.64. Test, using a tracking signal, to see if the forecast is unbiased (use 3 s.d. confidence intervals and a smoothing constant of 0.2).

Tracking signal range = +/- .55*3*sqrt(0.2) = +/- 0.7379

Initial Smoothed Absolute Error = 0.8 * Sqrt(.64) .64

Therefore forecast goes out of control in period 5

  - What are 3 advantages of U shaped flowlines?

Solution: includes better communications, use cross-trained labor, less travel.

  - What are the main steps in reducing setup time?

Solution: see text. Answer includes separating internal from external, convert activities from internal to external, reducing internal time, eliminate adjustments, abolish setup.

  - Calculate the minimum number of Kanbans required to avoid shortages in a workstation loop that has a demand rate of 400 per day, a lead time of 0.2 days, and a container size that holds 4 items. Assume a zero safety factor.

Solution: K= (DT)/N = (400*0.2)/ 4 = 30  K=20 Kanbans