Manufacturing Systems  	Quiz 1 Spring 2009    NAME ___________ ___________

SHOW WORKING TO RECEIVE FULL CREDIT

  - What is the weight of the demand 3 periods in the past in an exponential smoothed average with a smoothing constant of 0.2?

=alpha((1-alpha)^3)= .2*(1-.2)^3 = 0.1024

  - The demand for the new iGame, the multi-function game machine with built-in espresso maker, has been (in thousands) 102, 65, 132, 110 for weeks 1 to 4 this year respectively. Calculate the 3 week moving average forecast at week 4 for week 5

= (65+132+110)/3 = 102.33

  - A forecast has errors of 1.2, 0.3, -.8, -.3, 1.1. The forecast error variance (from past data) is 0.81. Test, using a tracking signal, to see if the forecast is unbiased (use 3 s.d. confidence intervals and a smoothing constant of 0.2).

Solution

Tracking signal range = +/- .55*3*sqrt(0.2) = +/- 0.7379

Forecast is unbiased because tracking signal is within range

- The quarterly sales of Budweiser NoTaste beer for the past 1 year are 107, 231, 303, 123 (millions). Using Winters method  (with all the exponential smoothing constants 0.3):

  - Calculate the initial parameters F(4) and c(1-4) using the approaches discussed in class and calculate the forecast for quarter 5, assuming that T(4) = 0

  - Calculate the updated parameters F, T, and c for quarter 5 and the forecast for quarter 6, if demand in quarter 5  is 112.

Solution

- Use exponential smoothing with trend (with both smoothing constants set to 0.2) to calculate a forecast for 1 period ahead for periods 1, 2 and 3.  Calculate the Mean Absolute Deviation for each forecast. Make sure that you show your working.

| Month | Demand | Smoothed Estimate | Smoothed Trend | Forecast | MAD |
| --- | --- | --- | --- | --- | --- |
| 0 |  | 82 | 0 |  |  |
| 1 | 93 |  |  |  |  |
| 2 | 77 |  |  |  |  |
| 3 | 102 |  |  |  |  |

Solution