Manufacturing Systems 		Quiz 3 Spring 2009   	NAME _________________________

Show ALL working for maximum credit

  - Briefly describe, using diagrams, the operation of a single card kanban system.

See class notes

  - How can work-in-process be systematically reduced using a kanban system

See class notes

  - What is a Kanban square?

See class notes

  - What does the term “poka-yoke” mean? Give an example.

See class notes

  - What are 4 advantages of U shaped flowlines?

See class notes

  - Calculate the minimum number of Kanbans required to avoid shortages in a workstation loop that has a demand rate of 500 per day, a lead time of 0.2 days, and a container size that holds 10 items. Assume a zero safety factor.

Solution: K= (DT)/N = (500*0.2)/ 10 = 10  K=10 Kanbans

  - A product has a desired probability of being able to assemble of 95%. If it is made up of 100 components, what is the required probability that an individual component be available (assuming that each component is independent)

Solution: S100=.95  S = .

  - What are the main data inputs for MRP

Solution: MPS, Item Master File (BoM etc), Inventory, Scheduled Receipts

  - Three jobs  are in line at work center YNOT. The operation times for A, B and C are 8, 21 and 9 (in hours) respectively, while the due date (days)  is 132, 152 and 141  respectively and the total remaining processing time (in days) is 12, 3.5 and 0.9. Today’s  date is day 106. Produce a priority list based on i) SPT ii) least slack

Solution:

    - SPT A, C, B

    - Slack =due date –remaining processing time – current time

A = 132- 12-106 = 14 ; B = 152-3.5-106  = 42.5;  C = 141 – 0.9 – 106 = 34.1

Priority list A, B, C

  - What are the main steps in reducing setup (changeover)  time reduction?

Solution: see text. Answer includes separating internal from external, convert activities from internal to external, reducing internal time, eliminate adjustments, abolish setup.