IE 516 Manufacturing Systems 		Exam I Spring 2008    	NAME _________________________

Failure to show working will reduce your grade.

- The monthly sales for the new iGame are as follows (in thousands):

| Period | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Demand | 72 | 76 | 92 |

Use exponential smoothing with linear trend and smoothing constants α = 0.3 and β=0.2. Assume that your starting smoothed estimate for month 1 is 72 and your smoothed trend estimate is 0.0. Calculate the forecast for periods 2, 3 and 4, calculated in periods 1, 2 and 3 respectively. Show your working for at least one calculation

Solution

- Briefly describe mixed model production using an example. See class notes

- What is the TAKT time?

See class notes

- An assembly requires thirty  components from 30 different suppliers. What must be the service level for each of the 30 components to start on-time assembly with 95% confidence?

S^30 = .95

S = .9983

- Briefly describe, using diagrams, the operation of a dual card kanban system.

See class notes

- How can work-in-process be systematically reduced using a kanban system

See class notes

- Items are consumed at a constant rate of 200 per month, with an ordering cost of $420 and a holding cost of $2 per item per month. The minimum reorder interval is one month and all order cycles are places on a power-of-2 multiple of months (1, 2 , 4, … months) in order to share truck capacity with other items.

  - What is the least cost reorder interval?

  - How much does it add to the total cost compared with using the EOQ?

Q* = sqrt(2AD/h) = sqrt( 2*420*200/2) Q*=290	 1.449 Months

Power of 2 : 	|	|	|	1.449 rounds to 2 months

1	1.41	4

½(T’/T* + T*/T’)= 0.5(2/1.449  +  1.449/2)  1.052  ie  5.2% cost increase

- The mean annual demand for an item is 60 units per year with an assumed a Poisson distribution, the holding cost is $35 per item per year, the replenishment lead time is 40 days, and the cost of a backorder is $140.  Calculate the reorder point (assuming a normal distribution of demand and using the critical ratio).

Demand over lead time θ = (60/365) * 40 = 6.576

σ = SQRT(6.576) = 2.564

b/(b+h) = 140/(140+35) = 0.8

Φ(0.8) = 0.84

r* = 6.576+ 0.84(2.564) = 8.730