Manufacturing Systems 		Quiz 2 Spring 2008   	NAME _________________________

SHOW WORKING TO RECEIVE FULL CREDIT

  - The demand for a part  follows a Poisson distribution with an average of 3 items per month. What is the probability of the demand being 2 or more in a month?

Prob of 2 or more  = 1 – 0.05 – 0.149 = 0.801

  - The demand for a part with a 1 month lead time follows a Poisson distribution with an average of 4 items per month. For mean = 4, the Poisson Distribution p(r) is shown in the table below. Calculate the cumulative distribution G(r) and the backorders B(r) for r = 0 to 4. Show your working for at least one of each calculation.

  - If the demand is as shown in 1b., the ordering cost is $35, the holding cost is $12 per item per year and the backorder cost is $96 per item per year, what is i) the optimal reorder quantity and ii) the optimal reorder point (assume that the demand is normally distributed)?

        - EOQ = sqrt(2AD/h) = sqrt(2x35x48/12) = 16.73 = 17

        - Mean = 4, sd = sqrt(4) = 2   critical ratio = b/(b+h) = 96/(96+12) = ..8888

Z from normal distribution tables  Φ(z) = 0.8888 z = 1.22

r* = mean + z x sd = 4 + 1.22 x 2 = 6.44

  - If the EOQ for a part is 100, what is the percentage increase in cost if 133 are ordered?

Cost ratio = 0.5(1.33+1/1.33)= 1.0409  Therefore there is a 4.1% increase in cost

- The demand for a part is 50 per month, a cost of $400, an ordering cost of $700 and a cost of capital of 10% per year.

  - What is the EOQ?

EOQ =Srt (2AD/h) = srt(2x700x600/40) = 144.913

  - The company decides to implement a  power-of-2  ordering policy with a minimum reorder interval of one month and all order cycles placed on a power-of-2 multiple of months (1, 2 , 4, … months) in order to share truck capacity with other items. What is the least cost reorder interval?

Order frequency is 144.913/50  = 2.89 since this is larger than 2*srt(2), the least cost reorder interval is 4 months