# Parametric Studies of Turbulent Flow around an Airfoil

# 57:020 Mechanics of Fluids and Transport Processes

# CFD LAB 2

By Tao Xing and Fred Stern

IIHR-Hydroscience & Engineering

The University of Iowa

C. Maxwell Stanley Hydraulics Laboratory

Iowa City, IA 52242-1585

1. Purpose

Geometry

Post-processing

Report

Physics

Mesh

Contours

Vectors

Streamlines

Solve

Iterations

Conver-gent.

Limit

Precisions

Single

Double

Numerical Schemes

1st  order

2nd order

QUICK

Steady/

Unsteady?

Coarse

Medium

Fine

Automatic

Manual

Structured

Unstruct-ured

Boundary

Conditions

Flow Properties

Viscous

Models

One Eq.

Two Eq.

Density and viscosity

Laminar

Turbulent

Inviscid

SA

k-e

k-w

Heat Transfer?

Incompress-ible?

Initial

Conditions

Clarky

NACA12

LS(1) 0417

Import Profile

Chord length

Angle of attack

Select domain

Select geometry

Wall shear stress

Skin friction Factor

XY plots

Verification and validation

Coefficient of lift

Coefficient of drag

Residuals

Pressure coef. Distri.

Shear stress Distri.

Airfoil Y plus

The Purpose of CFD Lab 2 is to conduct parametric studies for turbulent flow around Clarky airfoil following “CFD process” by an interactive step-by-step approach. Students will have “hands-on” experiences using FlowLab to investigate the effect of angle of attack and effect of different turbulence models on the simulations results. These effects will be studied by comparing simulation results with EFD data. Students will analyze the differences and possible numerical errors, and present results in Lab report.

Flow Chart for ISTUE Teaching Module for Airfoil flow (red color illustrates the options you will use in this CFD Lab 2)

### 2. Simulation Design

In EFD Lab 3, you have conducted experimental study for turbulent airfoil flow around a ClakY airfoil (Re=295,193). The data you have measured were used for CFD PreLab 2. In CFD Lab 2, simulation will be conducted under the same conditions of EFD Lab 3, except angle of attack and turbulent models which will be varied in this lab.

The problem to be solved is the turbulent flow around the Clarky airfoil with angle of attack ()

In the figure above, C is the chord length of the airfoil,  is the angle of attack, and Rc is the radius of the “O” domain.

3. CFD Process

Step 1: (Geometry)

Choose “ClarkY” airfoil and “O type” domain. Input the following conditions:

- Select Geometry  (Clarky)

- Select domain (O type)

- Chord length (0.3048 m)

- Angle of attack (refer to exercise 1 for instructions)

- Mesh type (Map)

- Circle Radius Rc (6m)

There are totally four angles of attack available in EFD Lab3 for all the groups, 0, 6, 13 and 16 degrees. In CFD PreLab2, you have conducted the simulation using your own EFD Lab3 data for just one angle of attack. In CFD Lab2, to investigate the “effect of angle attack”(exercise 1), you need choose one angle of attack that is different from the angle in your CFD PreLab2 (if your angle of attack in CFD PreLab2 is 13 or 16, you can use 0 degree for this lab, otherwise use 16 degrees). To investigate the “effect of turbulence models” (exercise 2), you will use the same angle of attack in your CFD PreLab2.

Step 2: (Physics)

(1). With or without Heat Transfer?

Since we are not dealing with the thermal aspects of the flow, like heat transfer, etc., switch the <<Heat Transfer >> button OFF, which is the default setup.

(2). Incompressible

Choose “Incompressible”, which is the default setup.

(3). Flow Properties

Use the air properties at the operating temperature of your EFD Lab3 and click <<OK>>. You can use the following website to calculate air properties if you know the temperature:

http://www.mhtl.uwaterloo.ca/old/onlinetools/airprop/airprop.html

NOTE: viscosity used in FlowLab is the dynamic viscosity (), NOT kinematic viscosity ()

(4). Viscous Model

To investigate the “effect of turbulence models” in exercise 2, using “k-w” model since k-e model has been used in CFD PreLab2. In exercise 1, use “k-e” model. Click <<OK>> after setup.

(5). Boundary Conditions

At “Inlet”, we use constant pressure. Inlet velocity should be computed from the EFD data reduction sheet and could be different from the value shown here. Use default values for “k”, “epsilon” and “omega(w)”.

k-e

k-w

At “Outlet”, FlowLab uses magnitude for pressure and zero gradients for axial and radial velocities. Input “0” for the Gauge pressure and click <<OK>>.

k-e

k-w

At “Airfoil”, if flow is turbulent, FlowLab uses no-slip boundary conditions for velocities and zero-pressure gradient. Turbulent quantities k and e are also specified to be zero, except for omega (w), on which zero-pressure gradient applied. Read all the values and click <<OK>>.

k-e

k-w

(6). Initial Conditions

Use the default setup for initial conditions.

k-e

k-w

After specifying all the above parameters, click <<Compute>> button and FlowLab will automatically calculate the Reynolds number based on the inlet velocity and airfoil chord length. Click <<Next>>, it takes you to the next step, “Mesh”.

Step 3: (Mesh)

For CFD Lab 2, use “Automatic” meshing and “Medium” mesh. After clicking <<Create>>, mesh information will be displayed. NA and NC are the numbers of grid points on the airfoil surface and radial direction, respectively.

Step 4: (Solve)

Specify the iteration number and convergence limit to be 10000 and 10-4, respectively. Choose “Double Precision”, “2nd order” for numerical schemes, and “New” calculation. Click <<Iterate>>, FlowLab will fire the XY plot for residuals that is dynamically updated during the calculation. Whenever you see the window, “Solution Converged”, click <<OK>>.

Step 5: (Reports)

“Reports” first provides you the information on “wall shear stress”, “skin friction factor”, “coefficient of lift”, and “coefficient of drag”. XY plot provide plots for “residuals”, “pressure distribution”, “pressure coefficient”, “friction coefficient”, “shear stress distribution”, and “Airfoil Y Plus distribution”. In this Lab, only XY plots for “residuals” and “pressure coefficient” are required.

“XY Plots” provides the following options:

To import the EFD data on top of the above CFD results, just click <<File>> button and use the browse button to locate the file you want and click <<Import>>.  Details have been described in previous CFD PreLab 1 for laminar pipe flow.

In this Lab:

- Since you already created your own EFD Lab 3 data in CFD PreLab 2, it is suggested that you use the same computer as you used in CFD PreLab2.

- For exercise 1, you will simulate another angle of attack which is different from the angle of attack you used in CFD PreLab2. TA data for that have been created and put in the following folder:

C:\Documents and Settings\fluidslab\myflowlab\TAdata_experiment_airfoil_2005\

The angle of attack can be told from the name of the file, such as “Pressure-coef-attack06.xy” is the EFD data of pressure coefficient for angle of attack equals 6 degrees.

Step 6: (Post-processing)

Use the “contour”, “vector” buttons to show pressure contours and velocity vectors. All the details on how to plot those figures have been explained in details in previous lab instructions. The following shows some sample results.

## 4. Exercises

# Parametric Studies of Turbulent Flow around an Airfoil

You must complete all the following assignments and present results in your CFD Lab 2 reports

following the CFD Lab Report Instructions.

- Effect of angle of attack.

1.1. Use the same flow conditions as those in your EFD Lab 3, including geometry (chord length) and physics (Flow properties, Reynolds number, inlet velocity), EXCEPT use an angle that is different from that in your CFD PreLab2 (if your angle of attack in CFD PreLab2 is 13 or 16, you can use 0 degree for this lab, otherwise use 16 degree). Use “O” mesh with domain size 6 meters, k-e model, automatic medium mesh, 2nd order upwind scheme, double precision with iteration number (10000) and convergent limit (10-4).

- Figures need to be saved: 1. Time history of residuals (residual vs. iteration number); 2.

Pressure coefficient distribution (CFD and EFD), 3. Contour of pressure, 4.Velocity vectors, and 5.

Streamlines

- Data need to be saved: lift and drag coefficients

- Effect of turbulence models

2.1. Use the same conditions as those in exercise 1, EXCEPT using the same angle of attack in your EFD Lab3, and “k-w” for “viscous models”. Set up the boundary conditions following instructions part, set the iteration number to be (10000), and convergent limit to be 10-4. Conduct the simulation and compare solutions with the simulation results using “k-e” model (you have finished in CFD PreLab2)

- Figures need to be saved: 1. Time history of residuals (residual vs. iteration number); 2. Pressure coefficient distribution (CFD and EFD), 3. Contour of pressure 4. Velocity vectors, and 5. Streamlines

- Data need to be saved: lift and drag coefficients

3.  Questions need to be answered in CFD Lab2 report:

Using the figures obtained in exercises 1 and 2 in this Lab and those figures you created in CFD

PreLab2 to answer the following questions and present your answers in your CFD Lab 2 report:

3.1. For Exercise 1 (effect of angle of attack):

(1). Which angle of attack simulation requires more iterations to converge?

(2). Which angle of attack produces higher lift/drag coefficients? Why?

(3). What is the effect of angle of attack on lift and drag coefficients?

(4). Describe the differences of streamline distributions near the trailing edge of airfoil surface for

these two different angles of attack. Do you observe separations for both?

3.2. For Exercise 2 (different turbulence models):

(1). Do the two different turbulence models have the same convergence path? If not, which one

requires more iterations to converge.

(2). Do the two different turbulence models predict the same results? If not, which model predicts

more accurately by comparing with EFD data?

3.3. For the following contour plot, qualitatively compare the values of pressure and velocity

magnitude at point 1 and 2, if the flow is from left to right. Which location has higher pressure

and which location has higher velocity magnitude? Why?

1

2

3.4. Questions in CFD PreLab2.