| NAME |  |
| --- | --- |
| Student ID |  |

Known: Water flows horizontally through a 1 inch inner diameter pipe over a length of 30 feet.  Water has the following properties: υ=1.21×10-5  ft2/s and ρ=1.94 slug/ft3.  The equivalent roughness of the pipe, ε=0.0001ft.

Find: Part 1: Determine the friction factor and the pressure drop if the average velocity is 0.2 ft/second.

Part 2:  Determine the friction factor and the pressure drop if the average velocity is 1 ft/second.

Assumptions: 1) The flow is fully developed. 2) No change in elevation. 3) There are no bends or contractions.

Equations:

Re = VD/υ

flam=64/Re

hL= f

l = 30 ft

D = 1 in

Solution:

Part 1:

Re   =   VD/ υ  =    (0.2 ft/s) (1/12 ft) / (0.0000121 ft2/s) = 1,377

- Laminar flow

flam = 64/Re = 64 / 1,377 = 0.0464

f  == = 0.649 = 0.0045

Part 2:

Re   =   VD/ υ  =    (1 ft/s) (1/12 ft) / (0.0000121 ft2/s) = 6887

- Turbulent flow

- Go to Moody diagram or Colebrook friction factor equation

ε/D = 0.0001ft / (1/12 ft) = 0.0012

fturb ~ 0.035

f  == = 12.2 = 0.0850