# 57:020

# Fluid Mechanics

# Class Notes

### Fall 2006

# Prepared by:

# Professor Fred Stern

# Typed by: Stephanie Schrader (Fall 1999)

# Corrected by: Jun Shao (Fall 2003)

# Corrected by: Jun Shao (Fall 2005)

# Corrected by: Jun Shao, Tao Xing (Fall 2006)

Corrected by: Hyunse Yoon (Fall 2007)

# Chapter 1: Introduction and basic concepts

Fluids and the no-slip condition

Fluid mechanics is the science of fluids either at rest (fluid statics) or in motion (fluid dynamics) and their effects on boundaries such as solid surfaces or interfaces with other fluids.

Definition of a fluid: a substance that deforms continuously when subjected to a shear stress

Consider a fluid between two parallel plates, which is subjected to a shear stress due to the impulsive motion of the upper plate

Fluid

Element







u=U

u=0

t=0

t=t

No slip condition:  no relative motion between fluid and boundary, i.e., fluid in contact with lower plate is stationary, whereas fluid in contact with upper plate moves at speed U.

Fluid deforms, i.e., undergoes strain  due to shear stress 

Newtonian fluid:	τ∝θ = rate of strain

τ = μ θ

 = coefficient of viscosity

Such behavior is different from solids, which resist shear by static deformation (up to elastic limit of material)





Solid



t=0

t=t

Elastic solid:     = strain

 = G 

G = shear modulus

Both liquids and gases behave as fluids

Liquids:

Closely spaced molecules with large intermolecular forces

Retain volume and take shape of container

container

liquid

Gases:

Widely spaced molecules with small intermolecular forces Take volume and shape of container

gas

Recall p-v-T diagram from thermodynamics:

single phase, two phase, triple point (point at which solid, liquid, and vapor are all in equilibrium), critical point (maximum pressure at which liquid and vapor are both in equilibrium).

Liquids, gases, and two-phase liquid-vapor behave as fluids.

Continuum Hypothesis

In this course, the assumption is made that the fluid behaves as a continuum, i.e., the number of molecules within the smallest region of interest (a point) are sufficient that all fluid properties are point functions (single valued at a point).

For example:

Consider definition of density  of a fluid

x = position vector

t = time

ρx,t=limδV→δVδMδV

V* = limiting volume below which molecular variations may be important and above which macroscopic variations may be important

V*  10-9 mm3 for all liquids and for gases at atmospheric pressure

10-9 mm3 air (at standard conditions, 20C and 1 atm) contains 3x107 molecules such that M/V = constant = 

Note that typical “smallest” measurement volumes are about 10-3 – 100 mm3 >> V* and that the “scale” of macroscopic variations are very problem dependent

Exception: rarefied gas flow

Properties of Fluids

Fluids are characterized by their properties such as viscosity  and density , which we have already discussed with reference to definition of shear stress τ = μθ and the continuum hypothesis.

Properties can be both dimensional (i.e., expressed in either SI or BG units) or non-dimensional.

See: Appendix Figures B.1 and B.2, and Appendix Tables B.1, B.2, B.3, B.4, and tables 1.3, 1.4, 1.5, 1.6, 1.7, and 1.8.

Basic Units

System International and British Gravitational Systems

| Primary Units | ## SI | BG |
| --- | --- | --- |
| Mass M | ## kg | Slug=32.2lbm |
| Length L | ## m | ft |
| Time t | s | s |
| Temperature T | C (K) | F (R) |

Temperature Conversion:

K = C + 273

R = F + 460

K and R are absolute scales, i.e., 0 at absolute zero.  Freezing point of water is at 0C and 32F.

| Secondary (derived) units | Dimension | SI | BG |
| --- | --- | --- | --- |
| velocity V | L/t | m/s | ft/s |
| acceleration a | L/t2 | m/s2 | ft/s2 |
| force F | ML/t2 | N (kgm/s2) | lbf |
| pressure p | F/L2 | Pa (N/m2) | lbf/ft2 |
| density  | M/L3 | kg/m3 | slug/ft3 |
| internal energy u | FL/M | J/kg (Nm/kg) | BTU/lbm |

Weight and Mass

Newton’s second law (valid for both solids

and fluids)

Weight = force on object due to gravity

W = mg		g = 9.81 m/s2

= 32.2 ft/s2

SI:	W (N) = M (kg)  9.81 m/s2

BG:	W (lbf) = 32.2 ft/s2 =M(slug)  32.2ft/ s2

gc = 32.2lbm⋅fts2⋅lbf = 32.2lbmslug, i.e., 1 slug = 32.2 lbm

1N = 1kg  1m/s2

1lbf = 1 slug  1ft/s2

System; Extensive and Intensive Properties

System = fixed amount of matter

= mass M

Therefore, by definition

d(M)dt = 0

Properties are further distinguished as being either extensive or intensive.

Extensive properties:	depend on total mass of system,

e.g., M and W (upper case letters)

Intensive properties:	independent of amount of mass of

system, e.g., p (force/area, lower case letters) and  (mass/volume)

Properties Involving the Mass or Weight of the Fluid

Specific Weight, 	=  gravitational force, i.e., weight per

unit volume

=  W/

=  mg/

=  g	N/m3

(Note that specific properties are extensive properties per unit mass or volume)

Mass Density  =  mass per unit volume

=  M/V	kg/m3

T = 4C

Specific Gravity S = ratio of fluid to water at standard					      = /water, 4C  dimensionless

water, 4C  = 9810 N/m3 for T = 4C and atmospheric pressure

Variation in Density

gases:    	=   (gas, T, p)  equation of state (p-v-T)

=  p/RT     ideal gas

R	=  R (gas)

R (air)  =  287.05  Nm/kgK

Air

liquids:   constant

| Liquid and temperature | Density (kg/m3) | Density (slugs/ft3) |
| --- | --- | --- |
| Water 20oC (68oF) | 998 | 1.94 |
| Ethyl alcohol 20oC (68oF) | 799 | 1.55 |
| Glycerine 20oC (68oF) | 1,260 | 2.45 |
| Kerosene 20oC (68oF) | 814 | 1.58 |
| Mercury 20oC (68oF) | 13,350 | 26.3 |
| Sea water 10oC at 3.3% salinity | 1,026 | 1.99 |
| SAE 10W 38oC(100oF) | 870 | 1.69 |
| SAE 10W-30 38oC(100oF) | 880 | 1.71 |
| SAE 30 38oC(100oF) | 880 | 1.71 |

# For greater accuracy can also use p-v-T diagram

 =  (liquid, T, p)

T		

p		

Properties Involving the Flow of Heat

For flows involving heat transfer such as gas dynamics additional thermodynamic properties are important, e.g.

specific heats			  cp and cv		J/kgK

specific internal energy		u		J/kg

specific enthalpy		 h = u + p/	 J/kg

Viscosity

# Recall definition of a fluid (substance that deforms continuously when subjected to a shear stress) and Newtonian fluid shear / rate-of-strain relationship (τ = μθ).

# Reconsider flow between fixed and moving parallel plates

(Couette flow)

ut=distance fluid particle travels in time t



f=fluid element

f at t

f at t

u=U

u=0

u(y)=velocity profile

y

y

h

Newtonian fluid: τ = μθ = μδθδt

tan δθ = δuδtδy or δθ = δuδtδy  for small 

therefore   δθ= δuδy		i.e.,	θ = dudy = velocity gradient

and		 τ = μdudy

Exact solution for Couette flow is a linear velocity profile

u(y) = Uhy 		Note: u(0) = 0 and u(h) = U

i.e., satisfies no-slip

boundary condition

τ = μUh= constant

where

U/h = velocity gradient = rate of strain

 = coefficient of viscosity = proportionality constant for

Newtonian fluid

ν=μρ=m2s = kinematic viscosity

 = (fluid;T,p) = (gas;T)

gas and liquid  p, but smal 

gas:			T

Due to structural differences, more molecular activity, decreased cohesive forces for gases

liquid:		T

Newtonian vs. Non-Newtonian Fluids

# Dilatant:				dV/dy

Newtonian:			dV/dy

Pseudo plastic:		dV/dy

Elasticity (i.e., compressibility)

Increasing/decreasing pressure corresponds to contraction/expansion of a fluid.  The amount of deformation is called elasticity.

dp=-EvdVV			dp>0⇒dVV<0

 minus sign used

EV=−dpdV/V=dpdρ/ρ=Nm2		EV=ρdpdρ

Alternate form:	M = V

dM = dV + Vd= 0  (by definition)

−dVV=dρρ

Liquids are in general incompressible, e.g.

Ev = 2.2 GN/m	2		 water

i.e.	V = .05% for p = 1MN/m2

(G=Giga=109 	  M=Mega=106	k=kilo=103)

Gases are in general compressible, e.g. for ideal gas at       T = constant (isothermal)

dpdρ=RT

Ev=ρRT=p

Vapor Pressure and Cavitation

When the pressure of a liquid falls below the vapor pressure it evaporates, i.e., changes to a gas.  If the pressure drop is due to temperature effects alone, the process is called boiling.  If the pressure drop is due to fluid velocity, the process is called cavitation.  Cavitation is common in regions of high velocity, i.e., low p such as on turbine blades and marine propellers.

high V  low p

(suction side)

isobars

streamlines around lifting surface (i.e. lines tangent to velocity vector)

low V  high p

(pressure side)

Cavitation number = p−pv12ρV∞2

< 0 implies cavitation

Surface Tension and Capillary Effects

Two non-mixing fluids (e.g., a liquid and a gas) will form an interface.  The molecules below the interface act on each other with forces equal in all directions, whereas the molecules near the surface act on each other with increased forces due to the absence of neighbors.  That is, the interface acts like a stretched membrane

# F = surface tension force

#### AIR

# F

F

Interface

Near surface forces are increased due to absence of neighbors such that surface is in tension  per unit length

Away from interface molecular forces are equal in all directions

#### WATER

air/water = 0.073 N/m

line force with direction normal to the cut

=length of cut through the interface

Effects of surface tension:

1. Capillary action in small tube		Δh=4σ/γd

2. Pressure difference across curved interface

p = /R			R = radius of curvature

3. Transformation of liquid jet into droplets

4. Binding of wetted granular material such as sand

# Example



F

F

h

d

Fluid attaches to solid with contact angle θ due to surface tension effect and wetty properties

water

reservoir

= contact angle

capillary tube d = 1.6mm = 0.0016m

, L=length of contact line between fluid & solid

water reservoir at 20 C,  = 0.073 N/m,   = 9790 N/m3

h = ?

Fz = 0

F,z - W = 0

d cos - gV = 0		0  cos = 1

g = 

spd−gDhpd24=0

=Volume of fluid above reservoir

Dh=4sgd=18.6mm

A brief history of fluid mechanics

See text book section 1.10.

Fluid Mechanics and Flow Classification

Hydrodynamics:  flow of fluids for which density is constant such as liquids and low-speed gases.  If in addition fluid properties are constant, temperature and heat transfer effects are uncoupled such that they can be treated separately.

Examples:  hydraulics, low-speed aerodynamics, ship hydrodynamics, liquid and low-speed gas pipe systems

Gas Dynamics:  flow of fluids for which density is variable such as high-speed gases.  Temperature and heat transfer effects are coupled and must be treated concurrently.

Examples:  high-speed aerodynamics, gas turbines,

high-speed gas pipe systems, upper atmosphere