# Chapter 2:  Pressure and Fluid Statics

Pressure

For a static fluid, the only stress is the normal stress since by definition a fluid subjected to a shear stress must deform and undergo motion.  Normal stresses are referred to as pressure p.

For the general case, the stress on a fluid element or at a point is a tensor

ij = stress tensor

=		xx	xy	xz

yx	yy	yz

zx	zy	zz

### i = force

j = direction

For a static fluid,

ij= 0 	ij		shear stresses = 0

ii= p = xx= yy= zz	i = j		normal stresses =-p

Also shows that p is isotropic, one value at a point which is independent of direction, a scalar.

Definition of Pressure:

N/m2 = Pa (Pascal)

F = normal force acting over A

As already noted, p is a scalar, which can be easily demonstrated by considering the equilibrium of forces on a wedge-shaped fluid element

Geometry

A =  y

x = cos

z = sin

### W = mg

= Vg

= V

V = ½ xzy

Fx = 0

pnA sin - pxA sin = 0

pn = px

Fz = 0

-pnA cos + pzA cos - W = 0

W=γ2(Δℓcosα)(Δℓsinα)Δy

x

z

−pn+pz−g2Dlsinα=0

pn=pzforDl→0

i.e.,    pn = px = py = pz

p is single valued at a point and independent of direction.

A body/surface in contact with a static fluid experiences a force due to p

Fp=−SBpndA

Note:  if p = constant, Fp = 0 for a closed body.

Scalar form of Green's Theorem:

f = constant f = 0

## Pressure Transmission

Pascal's law: in a closed system, a pressure change produced at one point in the system is transmitted throughout the entire system.

## Absolute Pressure, Gage Pressure, and Vacuum

pA < pa

pg < 0

pg > 0

pA > pa

pa = atmospheric

pressure =

101.325 kPa

pA = 0 = absolute

zero

For pA>pa,	pg = pA – pa = gage pressure

For pA<pa, 	pvac = -pg = pa – pA = vacuum pressure

Pressure Variation with Elevation

Basic Differential Equation

For a static fluid, pressure varies only with elevation within the fluid.  This can be shown by consideration of equilibrium of forces on a fluid element

1st order Taylor series estimate for pressure variation over dz

Newton's law (momentum principle) applied to a static fluid

F = ma = 0  for a static fluid

i.e., Fx = Fy = Fz = 0

Fz = 0

pdxdy−(p+¶p¶zdz)dxdy−ρgdxdydz=0			¶p¶z=-ρg=-g

Basic equation for pressure variation with elevation

Fy=0pdxdz−(p+∂p∂ydy)dxdz=0∂p∂y=0	Fx=0pdydz−(p+∂p∂xdx)dydz=0∂p∂x=0

For a static fluid, the pressure only varies with elevation z and is constant in horizontal xy planes.

The basic equation for pressure variation with elevation can be integrated depending on whether  = constant or             = (z), i.e., whether the fluid is incompressible (liquid or low-speed gas) or compressible (high-speed gas) since

g  constant

## Pressure Variation for a Uniform-Density Fluid

Z

g

¶p¶z=-ρg=-g

 = constant for liquid

Alternate forms:

constant

constant       piezometric pressure

gage

increase linearly with depth 
decrease linearly with height

i.e.,

constant     piezometric head

7.06

27.7

Pressure Variation for Compressible Fluids:

Basic equation for pressure variation with elevation

dpdz=−γ=−γ(p,z)=ρg

Pressure variation equation can be integrated for (p,z) known.  For example, here we solve for the pressure in the atmosphere assuming (p,T) given from ideal gas law, T(z) known, and g  g(z).

dry air

p = RT				R = gas constant = 287 J/kg K

p,T in absolute scale

dpdz=−pgRT

dpp=−gRdzT(z)		which can be integrated for T(z) known

# Pressure Variation in the Troposphere

T = To  (z – zo)		linear decrease

To = T(zo)	where p = po(zo) known

 = lapse rate = 6.5 K/km

dpp=−gRdz[To−α(z−zo)]		z'=To−α(z−zo)dz'=αdz

lnp=gαRln[To−α(z−zo)] + constant

zo = earth surface

= 0

po = 101.3 kPa

T = 15C

 = 6.5 K/km

use reference condition

lnpo=gαRlnTo + constant

solve for constant

lnppo=gαRlnTo−α(z−zo)Toppo=To−α(z−zo)Tog/αR

i.e., p decreases for increasing z

# Pressure Variation in the Stratosphere

T = Ts = 55C

dpp=-gRdzTs

lnp=−gRTsz + constant

use reference condition to find constant

ppo=e−(z−z0)gRTsp=poexp[−(z−zo)gRTs]

i.e., p decreases exponentially for increasing z.

Pressure Measurements

Pressure is an important variable in fluid mechanics and many instruments have been devised for its measurement.  Many devices are based on hydrostatics such as barometers and manometers, i.e., determine pressure through measurement of a column (or columns) of a liquid using the pressure variation with elevation equation for an incompressible fluid.

Differential manometer

More modern devices include Bourdon-Tube Gage (mechanical device based on deflection of a spring) and pressure transducers (based on deflection of a flexible diaphragm/membrane).  The deflection can be monitored by a strain gage such that voltage output is  p across diaphragm, which enables electronic data acquisition with computers.

Bourdon-Tube Gage

In this course we will use both manometers and pressure transducers in EFD labs 2 and 3.

# Manometry

- Barometer

pv + Hgh = patm

patm = Hgh		pv  0  i.e., vapor pressure Hg

nearly zero at normal T

h  76 cm

	patm  101 kPa (or 14.6 psia)

Note:	patm is relative to absolute zero, i.e., absolute pressure.  patm = patm(location, weather)

Consider why water barometer is impractical

γHghHg=γH2OhH2O

hH2O=γHgγH2OhHg=SHghHg=13.6 × 76= 1033.6 cm=34 ft.

- Piezometer

patm

patm + h = ppipe = p		absolute

p = h			gage

Simple but impractical for large p and vacuum pressures (i.e., pabs < patm). Also for small p and small d, due to large surface tension effects, could be corrected using , but accuracy may be problem if

3. U-tube or differential manometer



patm

p1 + mh  l = p4			p1 = patm

p4 = mh  l			gage

= w[Smh  S l]

for gases S << Sm and can be neglected, i.e., can neglect p in gas compared to p in liquid in determining p4 = ppipe.

Example:

Air at 20 C is in pipe with a water manometer.  For given conditions compute gage pressure in pipe.

air

l = 140 cm

h = 70 cm

p4 = ?    gage (i.e., p1 = 0)

Pressure same at 2&3 since same elevation & Pascal’s law: in closed system pressure change produce at one part transmitted throughout entire system

p1 + h = p3	step-by-step method

p3 - airl = p4

h

p1 + h - airl = p4		complete circuit method

h - airl = p4		gage

water(20C) = 9790 N/m3      p3 = h = 6853 Pa [N/m2]

air = g

pabs

or could use Table A.3

ρ=pRT=p3+patmR°C+273=6853+101300287(20+273)=1.286 kgm3

K

air = 1.286  9.81m/s2 = 12.62 N/m3

note  air << water

p4 = p3 - airl  = 6853 – 12.62  1.4 = 6835 Pa

17.668

if neglect effect of air column		p4 = 6853 Pa

A differential manometer determines the difference in pressures at two points ①and ② when the actual pressure at any point in the system cannot be determined.

p1γf+ℓ1−p2γf+ℓ2=γmγf−1Δh

difference in piezometric head

if fluid is a gas  f << m :  p1 – p2 = mh

if fluid is liquid & pipe horizontal  = :

p1 – p2 = (m - f) h

Hydrostatic Forces on Plane Surfaces

For a static fluid, the shear stress is zero and the only stress is the normal stress, i.e., pressure p.  Recall that p is a scalar, which when in contact with a solid surface exerts a normal force towards the surface.

Fp=−ApndA

For a plane surface n = constant such that we can separately consider the magnitude and line of action of Fp.

Fp=F=ApdA

Line of action is towards and normal to A through the center of pressure (xcp, ycp).

Unless otherwise stated, throughout the chapter assume patm acts at liquid surface.  Also, we will use gage pressure so that p = 0 at the liquid surface.

horizontal surface with area A

Horizontal Surfaces

p = constant

F

F=pdA=pA

Line of action is through centroid of A,

i.e., (xcp, ycp) = (x, y)

Inclined Surfaces

g

z

F

(x,y) = centroid of A

(xcp,ycp) = center of pressure

y

x

dF = pdA = y sin  dA

 and sin  are constants

y=1AydA

1st moment of area

p

F=ApdA=γsinαAydA

yA

F=γsinα y A

p= pressure at centroid of A

F=pA

Magnitude of resultant hydrostatic force on plane surface is product of pressure at centroid of area and area of surface.

# Center of Pressure

Center of pressure is in general below centroid since pressure increases with depth.  Center of pressure is determined by equating the moments of the resultant and distributed forces about any arbitrary axis.

Determine ycp by taking moments about horizontal axis 0-0

ycpF   = 	AydF

Ay pdA

Ay(γysinα)dA

=	γsinαAy2dA

Io = 2nd moment of area about 0-0

= moment of inertia

transfer equation:	Io=y2A+I

= 	moment of inertia with respect to horizontal

centroidal axis

ycpF=γsinα (y2A+I)ycp(pA)=γsinα (y2A+I)ycpγsinα yA=γsinα (y2A+I)

ycpyA=y2A+I

ycp is below centroid by IyA

ycp  y for large y

For po  0, y must be measured from an equivalent free surface located po/ above y.

Determine xcp by taking moment about y axis

xcpF    =	AxdF

AxpdA

xcpyA=AxydA

Ixy = product of inertia

= Ixy+xyA	transfer equation

xcpyA=Ixy+xyA

xcp=IxyyA+x

For plane surfaces with symmetry about an axis normal to 0-0, Ixy=0 and xcp = x.

Hydrostatic Forces on Curved Surfaces

Free surface

F=−ApndA

p = h

h = distance below free surface

# Horizontal Components		(x and y components)

Fx=F⋅i=−Apn⋅idA

dAx = projection of ndA onto plane  to x-direction

=−AxpdAx

Fy=F⋅j=−AypdAy	dAy=n⋅jdA

= projection ndA

onto plane  to

y-direction

Therefore, the horizontal components can be determined by some methods developed for submerged plane surfaces.

The horizontal component of force acting on a curved surface is equal to the force acting on a vertical projection of that surface including both magnitude and line of action.

# Vertical Components

Fz=F⋅k=−Apn⋅kdA

=−AzpdAz	p = h

h=distance

below free

surface

=γAzhdAz=γV

= weight of

fluid above

surface A

The vertical component of force acting on a curved surface is equal to the net weight of the column of fluid above the curved surface with line of action through the centroid of that fluid volume.

Example:  Drum Gate

Pressure Diagram

p = h = R(1-cos)

n=−sinθ i+cosθ k

dA =  Rd

F=−0πγR(1−cosθ)(−sinθ i+cosθ k)ℓRdθ

p			  n		  dA

F⋅i=Fx=+γ ℓ R20π(1−cosθ)sinθdθ

= γ ℓ R2[−cosθ+14cos2θ|0π=2γℓR2

= (R)(2R)  same force as that on projection of

p      A     		area onto vertical plane

Fz=−γ ℓ R20π(1−cosθ)cosθdθ

=−γ ℓ R2[sinθ−θ2−sin2θ4|0π

= γ ℓ R2π2=γ ℓπR22=γV

 net weight of water above surface

Buoyancy

## Archimedes Principle

FB  =  Fv2 – Fv1

= fluid weight above Surface 2 (ABC)

- fluid weight above Surface 1 (ADC)

= fluid weight equivalent to body volume V

FB = gV				V = submerged volume

Line of action is through centroid of V = center of buoyancy

Net Horizontal forces are zero since

FBAD = FBCD

## Hydrometry

A hydrometer uses the buoyancy principle to determine specific weights of liquids.

Stem

### Bulb

W = mg = fV = SwV

W = wV o = Sw(Vo  V) = Sw(Vo  ah)

f

V

a = cross section area stem

Vo/S = Vo  ah			ah = Vo – Vo/S

h = Voa⋅1−1S=h(S)

h = Voa⋅S−1S	calibrate scale using fluids of known S

S = VoV0−aΔh

Example (apparent weight)

King Hero ordered a new crown to be made from pure gold.  When he received the crown he suspected that other metals had been used in its construction.  Archimedes discovered that the crown required a force of 4.7# to suspend it when immersed in water, and that it displaced 18.9 in3 of water.  He concluded that the crown was not pure gold.  Do you agree?

Fvert = 0 = Wa + Fb – W = 0  Wa = W – Fb = (c - w)V

W=cV, 	 Fb = wV

or  c = WaV+γw=Wa+γwVV

γc=4.7+62.4×18.9172818.91728 = 492.1 = ρcg

   c = 15.3 slugs/ft3

 steel  and since gold is heavier than steel the crown

can not be pure gold

Stability of Immersed and Floating Bodies

Here we’ll consider transverse stability.  In actual applications both transverse and longitudinal stability are important.

## Immersed Bodies

Static equilibrium requires:  Fv=0 and M=0

M = 0 requires that the centers of gravity and buoyancy coincide, i.e., C = G and body is neutrally stable

If C is above G, then the body is stable (righting moment when heeled)

If G is above C, then the body is unstable (heeling moment when heeled)

## Floating Bodies

For a floating body the situation is slightly more complicated since the center of buoyancy will generally shift when the body is rotated depending upon the shape of the body and the position in which it is floating.

Positive GM		   Negative GM

The center of buoyancy (centroid of the displaced volume) shifts laterally to the right for the case shown because part of the original buoyant volume AOB is transferred to a new buoyant volume EOD.

The point of intersection of the lines of action of the buoyant force before and after heel is called the metacenter M and the distance GM is called the metacentric height.  If GM is positive, that is, if M is above G, then the ship is stable; however, if GM is negative, the ship is unstable.

## Floating Bodies

 = small heel angle

x=CC' = lateral displacement

of C

C = center of buoyancy

i.e., centroid of displaced

volume V

Solve for GM:  find x using

(1) basic definition for centroid of V; and

(2) trigonometry

Fig. 3.17

(1) Basic definition of centroid of volume V

xV=xd {V=xiΔVi	moment about centerplane

xV = moment V before heel – moment of VAOB

+ moment of VEOD

= 0 due to symmetry of

original V about y axis

i.e., ship centerplane

tan  = y/x

dV = ydA = x tan  dA

xV=tanαx2dA

ship waterplane area

moment of inertia of ship waterplane

about z axis O-O; i.e., IOO

IOO = moment of inertia of waterplane

area about centerplane axis

(2) Trigonometry

xV=tanαIOOCC'=x=tanαIOOV=CMtanα

CM = IOO / V

GM = CM – CG

GM = IOOV−CG

GM > 0		Stable

GM < 0		Unstable

Fluids in Rigid-Body Motion

For fluids in motion, the pressure variation is no longer hydrostatic and is determined from application of Newton’s 2nd Law to a fluid element.

ij = viscous stresses	      net surface force in X direction

p = pressure

Xnet=−∂p∂x+∂τxx∂x+∂τyx∂y+∂τzx∂zV

Ma = inertia force

W = weight (body force)

Newton’s 2nd Law				pressure		viscous

Ma = F = FB + FS

per unit volume ( V)   a = fb + fs

The acceleration of fluid particle

a = DVDt=∂V∂t+V⋅∇V

fb = body force = −ρgk

fs = surface force = fp + fv

fp = surface force due to p = p

fv = surface force due to viscous stresses ij

Neglected in this chapter and included later in Chapter 6 when deriving complete Navier-Stokes equations

inertia force = body force due   +  	surface force due to

to gravity			pressure gradients

Where for general fluid motion, i.e. relative motion between fluid particles:

substantial derivative

x:	ρDuDt=−∂p∂x

ρ∂u∂t+u∂u∂x+v∂u∂y+w∂u∂z=−∂p∂x

y:	ρDvDt=−∂p∂y

ρ∂v∂t+u∂v∂x+v∂v∂y+w∂v∂z=−∂p∂y

z:	ρDwDt=−ρg−∂p∂z=−∂∂zp+γz

Note: for V = 0

∇p=−ρgk∂p∂x=∂p∂y=0∂p∂z=−ρg=−γ

ρ∂w∂t+u∂w∂x+v∂w∂y+w∂w∂z=−∂p∂zp+γz

But in this chapter rigid body motion, i.e., no 
relative motion between fluid particles

a = (p + z)	Euler’s equation for inviscid flow

V = 0		Continuity equation for

incompressible flow (See Chapter 6)

4 equations in four unknowns V and p

For rigid body translation:

For rigid body rotating:

If , the motion equation reduces to hydrostatic equation:

Examples of Pressure Variation From Acceleration

Uniform Linear Acceleration:

ρa=−ρgk−∇p∇p=−ρa+gk=ρg−a g=−gk

∇p=−ρaxi+g+azk a=axi+azk

∂p∂x=−ρax ∂p∂z=−ρg+az

1. 		p increase in +x

2. 		p decrease in +x

1. 		p decrease in +z

2.  and 	p decrease in +z but slower than g

3.  and 	p increase in +z

s= unit vector in direction of p

=p /p

=−axi+g+azk ax2+g+az212

n = unit vector in direction of p = constant

= s×j			ijkijk

 to p

by definition lines of constant p are normal to p

= −axk+(g+az)i ax2+(g+az)212

 = tan-1 ax / (g + az) = angle between n and x

dpds=∇p⋅s=ρax2+g+az212	>  g

### G

p = Gs + constant    pgage = Gs

Rigid Body Rotation:

Consider a cylindrical tank of liquid rotating at a constant rate Ω=Ωk

a=Ω×Ω×ro

centripetal acceleration

= −rΩ2er

= −V2rer

∇p=ρ(g−a)				∇=∂∂rer+1r∂∂θeθ+∂∂zez

= −ρgk+ρrΩ2er	       grad in cylindrical coordinates

i.e., 	∂p∂r=ρrΩ2		∂p∂z=−ρg			∂p∂θ=0

C (r)

pressure distribution is hydrostatic in z direction

and	p = ρ2r2Ω2+f(z)+c

pz = -g

p = -gz + C(r) + c

p = ρ2r2Ω2−ρgz + constant		pγ+z−V22g=constant

V = r

The constant is determined by specifying the pressure at one point; say, p = po at (r, z) = (0, 0)

p = po  gz + 12r22

Note: pressure is linear in z and parabolic in r

Curves of constant pressure are given by

z = p1−poρg+r2Ω22g=a+br2

which are paraboloids of revolution, concave upward, with their minimum point on the axis of rotation

Free surface is found by requiring volume of liquid to be constant (before and after rotation)

The unit vector in the direction of p is

s=−ρgk+ρrΩ2er ρg2+ρrΩ2212

tanθ=dzdr=−grΩ2		slope of s

i.e.,  r = C1exp−Ω2zg		equation of p surfaces