3.4 Hydrostatic Forces on Plane Surfaces

For a static fluid, the shear stress is zero and the only stress is the normal stress, i.e., pressure p.  Recall that p is a scalar, which when in contact with a solid surface exerts a normal force towards the surface.

Fp=−ApndA

For a plane surface n = constant such that we can separately consider the magnitude and line of action of Fp.

Fp=F=ApdA

Line of action is towards and normal to A through the center of pressure (xcp, ycp).

Unless otherwise stated, throughout the chapter assume patm acts at liquid surface.  Also, we will use gage pressure so that p = 0 at the liquid surface.

horizontal surface with area A

Horizontal Surfaces

p = constant

F

F=pdA=pA

Line of action is through centroid of A,

i.e., (xcp, ycp) = (x, y)

Inclined Surfaces

g

z

F

(x,y) = centroid of A

dF = pdA = y sin  dA

(xcp,ycp) = center of pressure

y

x

 and sin  are constants

y=1AydA

1st moment of area

p

F=ApdA=γsinαAydA

yA

F=γsinα y A

p

= pressure at centroid of A

F=pA

Magnitude of resultant hydrostatic force on plane surface is product of pressure at centroid of area and area of surface.

# Center of Pressure

Center of pressure is in general below centroid since pressure increases with depth.  Center of pressure is determined by equating the moments of the resultant and distributed forces about any arbitrary axis.

Determine ycp by taking moments about horizontal axis 0-0

ycpF   = 	AydF

Ay pdA

Ay(γysinα)dA

=	γsinαAy2dA

Io = 2nd moment of area about 0-0

= moment of inertia

transfer equation:	Io=y2A+I

= 	moment of inertia with respect to horizontal

centroidal axis

ycpF=γsinα (y2A+I)ycp(pA)=γsinα (y2A+I)ycpγsinα yA=γsinα (y2A+I)

ycpyA=y2A+I

ycp is below centroid by IyA

ycp  y for large y

For po  0, y must be measured from an equivalent free surface located po/ above y.

Determine xcp by taking moment about y axis

xcpF    =	AxdF

AxpdA

xcpyA=AxydA

Ixy = product of inertia

= Ixy+xyA	transfer equation

xcpyA=Ixy+xyA

xcp=IxyyA+x

For plane surfaces with symmetry about an axis normal to 0-0, Ixy=0 and xcp = x.

3.5 Hydrostatic Forces on Curved Surfaces

Free surface

F=−ApndA

p = h

h = distance below free surface

# Horizontal Components		(x and y components)

Fx=F⋅i=−Apn⋅idA

dAx = projection of ndA onto plane  to x-direction

=−AxpdAx

Fy=F⋅j=−AypdAy	dAy=n⋅jdA

= projection ndA

onto plane  to

y-direction

Therefore, the horizontal components can be determined by some methods developed for submerged plane surfaces.

The horizontal component of force acting on a curved surface is equal to the force acting on a vertical projection of that surface including both magnitude and line of action.

# Vertical Components

Fz=F⋅k=−Apn⋅kdA

=−AzpdAz	p = h

h=distance

below free

surface

=γAzhdAz=γV

= weight of

fluid above

surface A

The vertical component of force acting on a curved surface is equal to the net weight of the column of fluid above the curved surface with line of action through the centroid of that fluid volume.

Example:  Drum Gate

Pressure Diagram

p = h = R(1-cos)

n=−sinθ i+cosθ k

dA =  Rd

F=−0πγR(1−cosθ)(−sinθ i+cosθ k)ℓRdθ

p			  n		  dA

F⋅i=Fx=+γ ℓ R20π(1−cosθ)sinθdθ

= γ ℓ R2[−cosθ+14cos2θ|0π=2γℓR2

= (R)(2R)  same force as that on projection of

p      A     		area onto vertical plane

Fz=−γ ℓ R20π(1−cosθ)cosθdθ

=−γ ℓ R2[sinθ−θ2−sin2θ4|0π

= γ ℓ R2π2=γ ℓπR22=γV

 net weight of water above surface

3.6 Buoyancy

## Archimedes Principle

FB  =  Fv2 – Fv1

= fluid weight above Surface 2 (ABC)

- fluid weight above Surface 1 (ADC)

= fluid weight equivalent to body volume V

FB = gV				V = submerged volume

Line of action is through centroid of V = center of buoyancy

Net Horizontal forces are zero since

FBAD = FBCD

## Hydrometry

A hydrometer uses the buoyancy principle to determine specific weights of liquids.

Stem

### Bulb

W = mg = fV = SwV

W = wV o = Sw(Vo  V) = Sw(Vo  ah)

f

V

a = cross section area stem

Vo/S = Vo  ah			ah = Vo – Vo/S

h = Voa⋅1−1S=h(S)

h = Voa⋅S−1S	calibrate scale using fluids of known S

S = VoV0−aΔh

Example (apparent weight)

King Hero ordered a new crown to be made from pure gold.  When he received the crown he suspected that other metals had been used in its construction.  Archimedes discovered that the crown required a force of 4.7# to suspend it when immersed in water, and that it displaced 18.9 in3 of water.  He concluded that the crown was not pure gold.  Do you agree?

Fvert = 0 = Wa + Fb – W = 0  Wa = W – Fb = (c - w)V

W=cV, 	 Fb = wV

or  c = WaV+γw=Wa+γwVV

γc=4.7+62.4×18.9172818.91728 = 492.1 = ρcg

   c = 15.3 slugs/ft3

 steel  and since gold is heavier than steel the crown

can not be pure gold

3.7 Stability of Immersed and Floating Bodies

Here we’ll consider transverse stability.  In actual applications both transverse and longitudinal stability are important.

## Immersed Bodies

Static equilibrium requires:  Fv=0 and M=0

M = 0 requires that the centers of gravity and buoyancy coincide, i.e., C = G and body is neutrally stable

If C is above G, then the body is stable (righting moment when heeled)

If G is above C, then the body is unstable (heeling moment when heeled)

## Floating Bodies

For a floating body the situation is slightly more complicated since the center of buoyancy will generally shift when the body is rotated depending upon the shape of the body and the position in which it is floating.

Positive GM		   Negative GM

The center of buoyancy (centroid of the displaced volume) shifts laterally to the right for the case shown because part of the original buoyant volume AOB is transferred to a new buoyant volume EOD.

The point of intersection of the lines of action of the buoyant force before and after heel is called the metacenter M and the distance GM is called the metacentric height.  If GM is positive, that is, if M is above G, then the ship is stable; however, if GM is negative, the ship is unstable.

## Floating Bodies

 = small heel angle

x=CC' = lateral displacement

of C

C = center of buoyancy

i.e., centroid of displaced

volume V

Solve for GM:  find x using

(1) basic definition for centroid of V; and

(2) trigonometry

Fig. 3.17

(1) Basic definition of centroid of volume V

xV=xd {V=xiΔVi	moment about centerplane

xV = moment V before heel – moment of VAOB

+ moment of VEOD

= 0 due to symmetry of

original V about y axis

i.e., ship centerplane

tan  = y/x

dV = ydA = x tan  dA

xV=tanαx2dA

ship waterplane area

moment of inertia of ship waterplane

about z axis O-O; i.e., IOO

IOO = moment of inertia of waterplane

area about centerplane axis

(2) Trigonometry

xV=tanαIOOCC'=x=tanαIOOV=CMtanα

CM = IOO / V

GM = CM – CG

GM = IOOV−CG

GM > 0		Stable

GM < 0		Unstable

3.8 Fluids in Rigid-Body Motion

For fluids in motion, the pressure variation is no longer hydrostatic and is determined from application of Newton’s 2nd Law to a fluid element.

ij = viscous stresses	      net surface force in X direction

p = pressure

Xnet=−∂p∂x+∂τxx∂x+∂τyx∂y+∂τzx∂zV

Ma = inertia force

W = weight (body force)

Newton’s 2nd Law				pressure		viscous

Ma = F = FB + FS

per unit ( V)   a = fb + fs

volume

a = DVDt=∂V∂t+V⋅∇V

fs = body force = −ρgk

fs = surface force = fp + fv

fp = surface force due to p = p

fv = surface force due to viscous stresses ij

ρDVDt=fb+fp+fv

Neglected in this chapter and included later in Section 6.4 when deriving complete Navier-Stokes equations

ρDVDt=−ρgk−∇p

inertia force = body force due   +  	surface force due to

to gravity			pressure gradients

x:	ρDuDt=−∂p∂x

Note: for V = 0

∇p=−ρgk∂p∂x=∂p∂y=0∂p∂z=−ρg=−γ

ρ∂u∂t+u∂u∂x+v∂u∂y+w∂u∂z=−∂p∂x

y:	ρDvDt=−∂p∂y

ρ∂v∂t+u∂v∂x+v∂v∂y+w∂v∂z=−∂p∂y

z:	ρDwDt=−ρg−∂p∂z=−∂∂zp+γz

ρ∂w∂t+u∂w∂x+v∂w∂y+w∂w∂z=−∂p∂zp+γz

or  a = (p + z)		Euler’s equation for inviscid flow

  V = 0			Continuity equation for

incompressible flow

4 equations in four unknowns V and p

Examples of Pressure Variation From Acceleration

Uniform Linear Acceleration:

ρa=−ρgk−∇p∇p=−ρa+gk=ρg−a g=−gk

∇p=−ρaxi+g+azk a=axi+azk

∂p∂x=−ρax ∂p∂z=−ρg+az

s= unit vector in direction of p

=p /p

=−axi+g+azk ax2+g+az212

n = unit vector in direction of p = constant

= s×j			ijkijk

 to p

by definition lines of constant p are normal to p

= −axk+(g+az)i ax2+(g+az)212

 = tan-1 ax / (g + az) = angle between n and x

dpds=∇p⋅s=ρax2+g+az212	>  g

### G

p = Gs + constant    pgage = Gs

Rigid Body Rotation:

Consider a cylindrical tank of liquid rotating at a constant rate Ω=Ωk		 =  in text

a=Ω×Ω×ro

centripetal acceleration

= −rΩ2er

= −V2rer

∇p=ρ(g−a)				∇=∂∂rer+1r∂∂θeθ+∂∂zez

= −ρgk+ρrΩ2er	       grad in cylindrical coordinates

i.e., 	∂p∂r=ρrΩ2		∂p∂z=−ρg			∂p∂θ=0

C (r)

along path of a = 0 pressure distribution is hydrostatic

and	p = ρ2r2Ω2+f(z)+c

pz = -g

p = -gz + C(r) + c

p = ρ2r2Ω2−ρgz + constant		pγ+z−V22g=constant

V = r

The constant is determined by specifying the pressure at one point; say, p = po at (r, z) = (0, 0)

p = po  gz + 12r22

Note: pressure is linear in z and parabolic in r

Curves of constant pressure are given by

z = p1−poρg+r2Ω22g=a+br2

which are paraboloids of revolution, concave upward, with their minimum point on the axis of rotation

Free surface is found by requiring volume of liquid to be constant (before and after rotation)

The unit vector in the direction of p is

s=−ρgk+ρrΩ2er ρg2+ρrΩ2212

tanθ=dzdr=−grΩ2		slope of s

i.e.,  r = C1exp−Ω2zg		equation of p surfaces