# Chapter 4:  Fluids Kinematics

  - Velocity and Description Methods

Primary dependent variable is fluid velocity vector

V = V ( r ); where r is the position vector

If V is known then pressure and forces can be determined using techniques to be discussed in subsequent chapters.

r=xi+yj+zk

V=(r,t)=ui+vj+wk

x

r

Consideration of the velocity field alone is referred to as flow field kinematics in distinction from flow field dynamics (force considerations).

Fluid mechanics and especially flow kinematics is a geometric subject and if one has a good understanding of the flow geometry then one knows a great deal about the solution to a fluid mechanics problem.

Consider a simple flow situation, such as an airfoil in a wind tunnel:

U = constant

Velocity: Lagrangian and Eulerian Viewpoints

There are two approaches to analyzing the velocity field: Lagrangian and Eulerian

Lagrangian:  keep track of individual fluids particles (i.e., solve F = Ma for each particle)

Say particle p is at position r1(t1) and at position r2(t2) then,

Vp=limΔt→0r2−r1t2−t1=dxdti+dydtj+dzdtk

= upi+vpj+wpk

Of course the motion of one particle is insufficient to describe the flow field, so the motion of all particles must be considered simultaneously which would be a very difficult task.  Also, spatial gradients are not given directly.  Thus, the Lagrangian approach is only used in special circumstances.

Eulerian:  focus attention on a fixed point in space

x=xi+yj+zk

In general,

V=V(x,t)=ui+vj+wk

velocity components

where,

u = u(x,y,z,t),   v = v(x,y,z,t),   w = w(x,y,z,t)

This approach is by far the most useful since we are usually interested in the flow field in some region and not the history of individual particles.

However, must transform F = Ma from system to CV (recall Reynolds Transport Theorem (RTT) & CV analysis from thermodynamics)

Ex. Flow around a car

V can be expressed in any coordinate system; e.g., polar or spherical coordinates.  Recall that such coordinates are called orthogonal curvilinear coordinates.  The coordinate system is selected such that it is convenient for describing the problem at hand (boundary geometry or streamlines).

V=vrer+vθeθ

x=rcosθy=rsinθer=cosθ i+sinθ jeθ=−sinθ i+cosθ j

Undoubtedly, the most convenient coordinate system is streamline coordinates:

V(s,t)=vs(s,t)es(s,t)

However, usually V not known a priori and even if known streamlines maybe difficult to generate/determine.

4.2 Flow Visualization and Plots of Fluid Flow Data

See textbook for:

Streamlines and Streamtubes

Pathlines

Streaklines

Timelines

Refractive flow visualization techniques

Surface flow visualization techniques

Profile plots

Vector plots

Contour plots

4.3 Acceleration Field and Material Derivative

The acceleration of a fluid particle is the rate of change of its velocity.

In the Lagrangian approach the velocity of a fluid particle is a function of time only since we have described its motion in terms of its position vector.

rp=xp(t)i+yp(t)j+zp(t)kVp=drpdt=upi+vpj+wpkap=dvpdt=d2rpdt2=axi+ayj+azkax=dupdt ay=dvpdt az=dwpdt

In the Eulerian approach the velocity is a function of both space and time; consequently,

V=u(x,y,z,t)i+v(x,y,z,t)j+w(x,y,z,t)k

x,y,z are f(t)

since we must follow the particle in evaluating du/dt

a=dVdt=dudti+dvdtj+dwdtk=axi+ayj+azk

ax=dudt=∂u∂t+∂u∂x∂x∂t+∂u∂y∂y∂t+∂u∂z∂z∂t=∂u∂t+u∂u∂x+v∂u∂y+w∂u∂z

called substantial derivative DuDt

Similarly for ay & az,

ay=DvDt=∂v∂t+u∂v∂x+v∂v∂y+w∂v∂zaz=DwDt=∂w∂t+u∂w∂x+v∂w∂y+w∂w∂z

In vector notation this can be written concisely

DVDt=∂V∂t+V⋅∇V

∇=∂∂xi+∂∂yj+∂∂zk  gradient operator

First term, ∂V∂t, called local or temporal acceleration results from velocity changes with respect to time at a given point.  Local acceleration results when the flow is unsteady.

Second term,V⋅∇V, called convective acceleration because it is associated with spatial gradients of velocity in the flow field.  Convective acceleration results when the flow is non-uniform, that is, if the velocity changes along a streamline.

The convective acceleration terms are nonlinear which causes mathematical difficulties in flow analysis; also, even in steady flow the convective acceleration can be large if spatial gradients of velocity are large.

Example:  Flow through a converging nozzle can be approximated by a one dimensional velocity distribution

u = u(x).  For the nozzle shown, assume that the velocity varies linearly from u = Vo at the entrance to u = 3Vo at the exit.  Compute the acceleration DVDt  as a function of x.  Evaluate DVDt at the entrance and exit if Vo = 10 ft/s and

y

L =1 ft.

u = Vo

## u(x) = mx + b

u(0) = b = Vo

m = ΔuΔx=3Vo−VoL=2VoL

We have	V=u(x)i,  DuDt=u∂u∂x=ax

u(x)=2VoLx+Vo=Vo2xL+1

Assume linear variation between inlet and exit

∂u∂x=2V0L⇒  ax=2Vo2L2xL+1

@ x = 0		ax = 200 ft/s2

@ x = L		ax = 600 ft/s2

Example problem: Deformation rate of fluid element

Consider the steady, two-dimensional velocity field given by

V=(u,v)=(0.5+0.8x)i+(1.5−0.8y)j

Calculate the kinematic properties such as;

- Rate of translation

- Rate of rotation

- Rate of linear strain

- Rate of shear strain

Solution:

(a) Rate of translation:

, ,

(b) Rate of rotation:

(c) Rate of linear strain:

, ,

(d) Rate of shear strain:

Rotation and Vorticity

 = fluid vorticity = 2  angular velocity = 2

=   V	i.e.,  curl V

=   i j k∂∂x ∂∂y ∂∂z u v w  =∂w∂y−∂v∂zi+∂u∂z−∂w∂xj+∂v∂x−∂u∂yk

To show that this definition is correct consider two lines in the fluid

Angular velocity about z axis = average rate of rotation +

ωz=12dαdt−dβdtdα=tan−1∂v∂xdxdtdx+∂u∂xdxdt

limdt→0=∂v∂xdt			i.e.,  dαdt=∂v∂x

dβ=tan−1∂u∂ydydtdy+∂v∂ydydt

limdt→0=∂u∂ydt			i.e.,  dβdt=∂u∂y

similarly,	   ωz=12∂v∂x−∂u∂yωx=12∂w∂y−∂v∂zωy=12∂u∂z−∂w∂x

i.e.,   = 2

Example problem: Calculation of Vorticity

Consider the following steady, three-dimensional velocity field

Calculate the vorticity vector as a function of space

Solution:

Vorticity vector in Cartesian coordinates:

For , ,