Chapter 5 Mass, Bernoulli, and Energy Equations

5.1 Flow Rate and Conservation of Mass

- cross-sectional area oriented normal to velocity vector

(simple case where V  A)

U = constant:  Q = volume flux = UA [m/s  m2 = m3/s]

U  constant:  Q = AUdA

Similarly the mass flux = m=AρUdA

- general case

Q=CSV⋅ndA =CSVcosθdA

m=CSρV⋅ndA

average velocity:  V=QA

Example:

At low velocities the flow through a long circular tube, i.e. pipe, has a parabolic velocity distribution (actually paraboloid of revolution).

u=umax1−rR2

i.e., centerline velocity

a) find Q and V

Q=AV⋅ndA=AudA

AudA=02π0Ru(r)rdθdr

= 2π0Ru(r)rdr

dA = 2rdr

u = u(r) and not   02πdθ=2π

Q = 2π0Rumax1−rR2rdr  = 12umaxπR2

V=12umax

Continuity Equation

RTT can be used to obtain an integral relationship expressing conservation of mass by defining the extensive property B = M such that  = 1.

B = M = mass

 = dB/dM = 1

General Form of Continuity Equation

dMdt=0=ddtCVρdV+CSρV⋅dA

or

CSρV⋅dA = −ddtCVρdV

net rate of outflow 		rate of decrease of

of mass across CS		mass within CV

Simplifications:

- Steady flow:  −ddtCVρdV=0

- V = constant over discrete dA (flow sections):

CSρV⋅dA=CSρV⋅A

- Incompressible fluid ( = constant)

conservation of volume

- Steady One-Dimensional Flow in a Conduit:

CSρV⋅A=0

1V1A1 + 2V2A2 = 0

for  = constant	Q1 = Q2

Some useful definitions:

Mass flux			m=AρV⋅dA

Volume flux		Q=AV⋅dA

Average Velocity	V=QA

Average Density	ρ=1AρdA

Note:   unless  = constant

Example

*Steady flow

*V1,2,3 = 50 fps

*@  V varies linearly

from zero at wall to

Vmax at pipe center

*find m4, Q4, Vmax

0		*water, w = 1.94 slug/ft3

CSρV⋅dA=0=−ddtCVρdV

m4

i.e.,  -1V1A1 - 2V2A2 + 3V3A3 + A4V4dA4= 0

 = const. = 1.94 lb-s2 /ft4 = 1.94 slug/ft3

m4=ρV4dA4= V(A1 + A2 – A3)      V1=V2=V3=V=50f/s

= 1.94144×50×π412+22−1.52

= 1.45 slugs/s

Q4 = m4/ρ=.75 ft3/s

= A4V4dA4

velocity profile

Q4   = 0ro02πVmax1−rrordθdr

dA4

V4  V4()

=2π0roVmax1−rrordr=2πVmax0ror−r2rodr=2πVmaxr22|0r0−r33ro|0ro=2πVmaxro212−13=13πro2Vmax

V4=QA=13πro2Vmaxπro2

= 13Vmax

Vmax = Q413πro2=2.86fps

5.2 Mechanical Energy, Efficiency, Bernoulli Equations, Application, and Limitations

Assume irrotational, inviscid, and incompressible flow = ideal flow theory

Also, assume steady flow

 =   V = 0    V = 		irrotational

a =  (p/ + gz),    V = 0 		inviscid, incompressible

0

a = V V = ½V  V + V  (  V)		steady

= ½V2		V2 = V  V

∇12V2=−∇pρ+gz

∇12V2+pρ+gz=0

i.e.,  p + ½V2 + z = B = constant

p1 + ½V12 + z1 = p2 + ½V22 + z2

Also, from continuity and irrotational

  V = 0			V =  = ∂ϕ∂xi+∂ϕ∂yj+∂ϕ∂zk

   = 0		 = velocity potential

2 = 0			i.e., governing differential

equation for  is Laplace equation

Application of Bernoulli’s Equation

Stagnation Tube

at   V = 0

p1+ρV122=p2+ρV222			z1 = z2

V12=2ρp2−p1			p1=γdp2=γℓ+d

V2 = 0

gage

= 2ργℓ

Limited by length of tube and need for free surface reference

V1=2gℓ

Pitot Tube

0

p1γ+V122g+z1=p2γ+V222g+z2

at 		       at 

V2=2gp1γ+z1−p2γ+z212		V1 = 0

h1		  h2

h = piezometric head

V=V2=2gh1−h2			h1 – h2 from manometer

or pressure gage

for gas flow Δpγ>>Δz

V=2Δpρ

5.3 Derivation of the Energy Equation

The First Law of Thermodynamics

The difference between the heat added to a system and the work done by a system depends only on the initial and final states of the system; that is, depends only on the change in energy E: principle of conservation of energy

E = Q – W

E = change in energy

Q = heat added to the system

W = work done by the system

E = Eu + Ek + Ep = total energy of the system

potential energy

kinetic energy

Internal energy due to molecular motion

The differential form of the first law of thermodynamics expresses the rate of change of E with respect to time

dEdt=Q−W

rate of work being done by system

rate of heat transfer to system

Energy Equation for Fluid Flow

The energy equation for fluid flow is derived from Reynolds transport theorem with

Bsystem = E = total energy of the system (extensive property)

 = E/mass = e = energy per unit mass (intensive property)

= u + ek + ep

dEdt=ddtρed {V+ρeV⋅dA

Q−W=ddtρ(u+ek+ep)dV+ρ(u+ek+ep)V⋅dA

This can be put in a more useable form by noting the following:

ek=Total KE of mass with velocity Vmass=ΔMV22ΔM=V22

V2=V

ep=EpΔM=γΔVzρΔV=gz		(for Ep due to gravity only)

Q−W=ddtρV22+gz+udV+ρV22+gz+uV⋅dA

rate of work			rate of change		flux of energy

done by system		of energy in CV		out of CV

(ie, across CS)

rate of heat

transfer to sysem

Rate of Work Components:  W=Ws+Wf

For convenience of analysis, work is divided into shaft work Ws and flow work Wf

Wf = net work done on the surroundings as a result of

normal and tangential stresses acting at the control

surfaces

= Wf pressure + Wf shear

Ws = any other work transferred to the surroundings

usually in the form of a shaft which either takes

energy out of the system (turbine) or puts energy into

the system (pump)

Flow work due to pressure forces  Wf p  (for system)

Note: here V uniform over A

System at time t + t

System at time t

CS

CV

Work = force  distance

at 2	W2 = p2A2  V2t

(on surroundings)

rate of work W2=p2A2V2=p2V2⋅A2

at 1	W1 = p1A1  V1t

neg. sign since pressure force on surrounding fluid acts in a direction opposite to the motion of the system boundary

W1=p1V1⋅A1

In general,

Wfp=pV⋅A

for more than one control surface and V not necessarily uniform over A:

Wfp=pV⋅dA=ρpρV⋅dA

Wf=Wfp+Wfshear

### Basic form of energy equation

Q−Ws−Wfshear−ρpρV⋅dA =ddtρV22+gz+udV+ρV22+gz+uV⋅dA

Q−Ws−Wfshear=ddtρV22+gz+udV +ρV2+gz+u+pρV⋅dA  									h=enthalpy

#### Usually this term can be eliminated by proper choice of CV, i.e. CS normal to flow lines.  Also, at fixed boundaries the velocity is zero (no slip condition) and no shear stress flow work is done.  Not included or discussed in text!

5.4 Simplified Forms of the Energy Equation

Energy Equation for Steady One-Dimensional Pipe Flow

Consider flow through the pipe system as shown

Energy Equation (steady flow)

Q−Ws=ρV22+gz+pρ+uV⋅dA

Q−Ws+p1ρ+gz1+u1ρ1V1A1+ρ1V132dA1 =p2ρ+gz2+u2ρ2V2A2+ρ2V232dA2

*Although the velocity varies across the flow sections the streamlines are assumed to be straight and parallel; consequently, there is no acceleration normal to the streamlines and the pressure is hydrostatically distributed, i.e., p/ +gz = constant.

*Furthermore, the internal energy u can be considered as constant across the flow sections, i.e. T = constant.  These quantities can then be taken outside the integral sign to yield

Q−Ws+p1ρ+gz1+u1ρV1dA1+ρV132dA1=p2ρ+gz2+u2ρV2dA2+ρV232dA2

Recall that	Q=V⋅dA=VA

So that		ρV⋅dA=ρVA=m		mass flow rate

Define:		ρAV32dA=αρV3A2=αV22m

K.E. flux              K.E. flux for V=V=constant across pipe

i.e.,	      α=1AAVV3dA = kinetic energy correction factor

Q−W+p1ρ+gz1+u1+α1V122m=p2ρ+gz2+u2+α2V222m1mQ−W+p1ρ+gz1+u1+α1V122=p2ρ+gz2+u2+α2V222

note that:		 = 1 if V is constant across the flow section

 > 1 if V is nonuniform

laminar flow  = 2	       turbulent flow  = 1.05   1 may

be used

Shaft Work

Shaft work is usually the result of a turbine or a pump in the flow system.  When a fluid passes through a turbine, the fluid is doing shaft work on the surroundings; on the other hand, a pump does work on the fluid

Ws=Wt−Wp		where Wt and Wp are

magnitudes of power	worktime

Using this result in the energy equation and deviding by g results in

Wpmg+p1γ+z1+α1V122=Wtmg+p2γ+z2+α2V222+u2−u1g−Qmg

mechanical part			    thermal part

Note: each term has dimensions of length

Define the following:

hp=Wpmg=WpρQg=WpγQ

ht=Wtmg

hL=u2−u1g−Qmg=head loss

Head Loss

In a general fluid system a certain amount of mechanical energy is converted to thermal energy due to viscous action.  This effect results in an increase in the fluid internal energy.  Also, some heat will be generated through energy dissipation and be lost (i.e. -Q).  Therefore the term

from 2nd law

represents a loss in mechanical energy due to viscous stresses

hL=u2−u1g−Qgm>0

Note that adding Q to system will not make hL = 0 since this also increases u.  It can be shown from 2nd law of thermodynamics that hL > 0.

Drop  over V and understand that V in energy equation refers to average velocity.

Using the above definitions in the energy equation results in (steady 1-D incompressible flow)

p1γ+α1V122g+z1+hp=p2γ+α2V222g+z2+ht+hL

form of energy equation used for this course!

Comparison of Energy Equation and Bernoulli Equation:

Apply energy equation to a stream tube without any shaft work

Infinitesimal stream tube Þ 1=2=1

Energy eq :	p1γ+V122g+z1=p2γ+V222g+z2+hL

If hL = 0 (i.e.,  = 0) we get Bernoulli equation and conservation of mechanical energy along a streamline

Therefore, energy equation for steady 1-D pipe flow can be interpreted as a modified Bernoulli equation to include viscous effects (hL) and shaft work (hp or ht)

5.5 Concept of Hydraulic and Energy Grade Lines

p1γ+α1V122g+z1+hp=p2γ+α2V222g+z2+ht+hL

Define 	HGL = pγ+z

point-by-point application is graphically displayed

EGL = pγ+z+αV22g

# HGL corresponds to pressure tap measurement + z

# EGL corresponds to stagnation tube measurement + z

EGL = HGL if V = 0

hL = fLDV22g

i.e., linear variation in L for D,

V, and f constant

EGL1 = EGL2 + hL

for hp = ht = 0

f = friction factor

f = f(Re)

pressure tap:  p2γ=h

h = height of fluid in

tap/tube

stagnation tube:  p2γ+αV222g=h

EGL1 + hp = EGL2 + ht + hL

EGL2 = EGL1 + hp  ht  hL

abrupt change due to hp or ht

fLDV22g

Helpful hints for drawing HGL and EGL

- EGL = HGL + V2/2g = HGL for V = 0

2.&3. hL=fLDV22g in pipe means EGL and HGL will slope

downward, except for abrupt changes due to ht or hp

p1γ+z1+V122g=p2γ+z2+V222g+hL

HGL2 = EGL1 - hL

hL=V22gfor abrupt expansion

4. p = 0  HGL = z

5. for hL=fLDV22g = constant  L

i.e., linearly increased for increasing L with slope fDV22g

EGL/HGL slope downward

6. for change in D  change in V

i.e.	V1A1 = V2A2

change in distance between HGL & EGL and slope

change due to change in hL

V1πD124=V2πD224



V1D12=V1D22

7.  If HGL < z then p/ < 0 	i.e., cavitation possible

condition for cavitation:

p=pva=2000Nm2

gage pressure	pva,g=pA−patm≈−patm=−100,000Nm2

pva,gγ≈−10m

9810 N/m3

Summary of the Energy Equation

The energy equation is derived from Reynolds Transport Theorem with

B = E = total energy of the system

 = e = E/M  = energy per unit mass

= u + 12V2+gz

internal	  KE	     PE

dEdt=ddtCVρed {V+CSρeV⋅dA=Q−W

heat addp1γ+z1+V122ged

work done

from 1st Law of Thermodynamics

##### Neglected in text presentation

W=Ws+Wp+Wv

shaft work done on or by system (pump or turbine)

pressure work done on CS

Viscous stress work on CS

Wp=CVpV⋅dA=CSρp/ρV⋅dA

Ws=Wt−Wp

Q−Wt+Wp=ddtCVρed {V+CSρe+p/eV⋅dA

e=u+12V2+gz

For steady 1-D pipe flow (one inlet and one outlet):

- Streamlines are straight and parallel

 p/ +gz = constant across CS

- T = constant  u = constant across CS

3)	define	α=1ACSVV3dA = KE correction factor

	ρ2V3dA=αρV32A=αV22m

mechanical energy

Thermal energy

p1γ+α1V122g+z1+hp=p2γ+α2V222g+z2+ht+hL

# Note: each term has

units of length

V is average velocity (vector dropped) and

corrected by 

hp=Wp/mg

ht=Wt/mg

hL=u2−u1g−Qmg= head loss

> 0  represents loss in mechanical energy due to viscosity