# Chapter 5 Mass, Momentum, and Energy Equations

Flow Rate and Conservation of Mass

- cross-sectional area oriented normal to velocity vector

(simple case where V  A)

U = constant:  Q = volume flux = UA [m/s  m2 = m3/s]

U  constant:  Q = AUdA

Similarly the mass flux = m=AρUdA

- general case

Q=CSV⋅ndA =CSVcosθdA

m=CSρV⋅ndA

average velocity:  V=QA

Example:

At low velocities the flow through a long circular tube, i.e. pipe, has a parabolic velocity distribution (actually paraboloid of revolution).

u=umax1−rR2

i.e., centerline velocity

a) find Q and V

Q=AV⋅ndA=AudA

AudA=02π0Ru(r)rdθdr

= 2π0Ru(r)rdr

dA = 2rdr

u = u(r) and not   02πdθ=2π

Q = 2π0Rumax1−rR2rdr  = 12umaxπR2

V=12umax

Continuity Equation

RTT can be used to obtain an integral relationship expressing conservation of mass by defining the extensive property B = M such that  = 1.

B = M = mass

 = dB/dM = 1

General Form of Continuity Equation

dMdt=0=ddtCVρdV+CSρV⋅dA

or

CSρV⋅dA = −ddtCVρdV

net rate of outflow 		rate of decrease of

of mass across CS		mass within CV

Simplifications:

- Steady flow:  −ddtCVρdV=0

- V = constant over discrete dA (flow sections):

CSρV⋅dA=CSρV⋅A

- Incompressible fluid ( = constant)

conservation of volume

- Steady One-Dimensional Flow in a Conduit:

CSρV⋅A=0

1V1A1 + 2V2A2 = 0

for  = constant	Q1 = Q2

Some useful definitions:

Mass flux			m=AρV⋅dA

Volume flux		Q=AV⋅dA

Average Velocity	V=QA

Average Density	ρ=1AρdA

Note:   unless  = constant

Example

*Steady flow

*V1,2,3 = 50 fps

*@  V varies linearly

from zero at wall to

Vmax at pipe center

*find m4, Q4, Vmax

0		*water, w = 1.94 slug/ft3

CSρV⋅dA=0=−ddtCVρdV

m4

i.e.,  -1V1A1 - 2V2A2 + 3V3A3 + A4V4dA4= 0

 = const. = 1.94 lb-s2 /ft4 = 1.94 slug/ft3

m4=ρV4dA4= V(A1 + A2 – A3)      V1=V2=V3=V=50f/s

= 1.94144×50×π412+22−1.52

= 1.45 slugs/s

Q4 = m4/ρ=.75 ft3/s

= A4V4dA4

velocity profile

Q4   = 0ro02πVmax1−rrordθdr

dA4

V4  V4()

=2π0roVmax1−rrordr=2πVmax0ror−r2rodr=2πVmaxr22|0r0−r33ro|0ro=2πVmaxro212−13=13πro2Vmax

V4=QA=13πro2Vmaxπro2

= 13Vmax

Vmax = Q413πro2=2.86fps

Momentum Equation

RTT with B = MV and  = V

FS+FB=ddtCVρVdV+CSVρVR⋅dA

V = velocity referenced to an inertial frame (non-accelerating)

VR = velocity referenced to control volume

FS = surface forces + reaction forces (due to pressure and

viscous normal and shear stresses)

FB = body force (due to gravity)

Applications of the Momentum Equation

Initial Setup and Signs

- Jet deflected by a plate or a vane

- Flow through a nozzle

- Forces on bends

- Problems involving non-uniform velocity distribution

- Motion of a rocket

- Force on rectangular sluice gate

- Water hammer

General form for moving but

non-accelerating reference frame

Derivation of the Basic Equation

Recall RTT:	dBsysdt=ddtCVβρdV+CSβρVR⋅dA

VR=velocity relative to CS=V – VS=absolute – velocity CS

Subscript not shown in text but implied!

i.e., referenced to CV

Let, 		B = MV = linear momentum

V must be referenced to inertial reference frame

 = V

d(MV)dt=F=ddtCVVρdV+CSVρVR⋅dA

Must be relative to a non-accelerating inertial reference frame

Newton’s 2nd law

where	F = vector sum of all forces acting on the

control volume including both surface and

body forces

= FS + FB

FS = sum of all external surface forces acting at

the CS, i.e., pressure forces, forces

transmitted through solids, shear forces, etc.

FB = sum of all external

body forces, i.e.,

gravity force

Fx = p1A1 – p2A2 + Rx

Fy = -W + Ry

R = resultant force on fluid

in CV due to pw and w

free body diagram

i.e., reaction force on fluid

Important Features (to be remembered)

- Vector equation to get component in any direction must use dot product

Carefully define coordinate system with forces positive in positive direction of coordinate axes

x equation

Fx=ddtCVρud {V+CSρuVR⋅dA

y equation

Fy=ddtCVρvd {V+CSρvVR⋅dA

z equation

Fz=ddtCVρwd {V+CSρwVR⋅dA

- Carefully define control volume and be sure to include all external body and surface faces acting on it.

For example,

(Rx,Ry) = reaction force on fluid

(Rx,Ry) = reaction force on nozzle

- Velocity V must be referenced to a non-accelerating inertial reference frame.  Sometimes it is advantageous to use a moving (at constant velocity) reference frame.  Note VR = V – Vs is always relative to CS.

i.e., in these cases V used for B also referenced to CV (i.e., V = VR)

- Steady vs. Unsteady Flow

Steady flow  ddtCVρVdV=0

- Uniform vs. Nonuniform Flow

CSVρVR⋅dA = change in flow of momentum across CS

= VVRA		uniform flow across A

- Fpres = pndA			V∇fd {V=Sfnds

f = constant, f = 0

= 0  for p = constant and for a closed surface

i.e., always use gage pressure

- Pressure condition at a jet exit

at an exit into the atmosphere jet pressure must be pa

Application of the Momentum Equation

1.  Jet deflected by a plate or vane

Consider a jet of water turned through a horizontal angle

CV and CS are for jet so that Fx and Fy are vane reactions forces on fluid

x-equation:	Fx=Fx=ddtρud {V+CSρuV⋅dA

Fx=CSρuV⋅A								steady flow

= ρV1x(−V1A1)+ρV2x(V2A2)

continuity equation:	A1V1 = A2V2 = Q

for A1 = A2

V1 = V2

Fx = Q(V2x – V1x)

y-equation:	Fy=Fy=CSρvV⋅A

Fy = V1y(– A1V1) + V2y(– A2V2)

= Q(V2y – V1y)

for above geometry only

where: 	V1x = V1    V2x = -V2cos   V2y = -V2sin  V1y = 0

note:		Fx and Fy are force on fluid

- Fx and -Fy are force on vane due to fluid

If the vane is moving with velocity Vv, then it is convenient to choose CV moving with the vane

i.e.,	 VR = V -  Vv  and V used for B also moving with vane

x-equation:	Fx=CSρuVR⋅dA

Fx = V1x[-(V – Vv)1A1] + V2x[-(V – Vv)2A2]

Continuity:	0 = ρVR⋅dA

i.e.,	(V-Vv)1A1 = (V-Vv)2A2 = (V-Vv)A

Qrel

Fx = (V-Vv)A[V2x – V1x]

Qrel

on fluid		V2x = (V – Vv)2x

For coordinate system moving with vane

V1x = (V – Vv)1x

Power = -FxVv			i.e., = 0 for Vv = 0

Fy = Qrel(V2y – V1y)

- Flow through a nozzle

Consider a nozzle at the end of a pipe (or hose).  What force is required to hold the nozzle in place?

### CV = nozzle

and fluid

 (Rx, Ry) = force required to hold nozzle in place

Assume either the pipe velocity or pressure is known.  Then, the unknown (velocity or pressure) and the exit velocity V2 can be obtained from combined use of the continuity and Bernoulli equations.

Bernoulli:	p1+γz1+12ρV12=p2+γz2+12ρV22      z1=z2

p1+12ρV12=12ρV22

Continuity:	A1V1 = A2V2 = Q

V2=A1A2V1=Dd2V1

p1+12ρV121−Dd4=0

Say p1 known:		V1=−2p1ρ1−Dd412

To obtain the reaction force Rx apply momentum equation in x-direction

Fx=ddtCVuρdV+CSρuV⋅dA

steady flow and uniform

flow over CS

=CSρuV⋅A

Rx + p1A1 – p2A2  	= V1(-V1A1) + V2(V2A2)

= Q(V2 - V1)

Rx	= Q(V2 - V1) - p1A1

To obtain the reaction force Ry apply momentum equation in y-direction

Fy=CSρvV⋅A=0	since no flow in y-direction

Ry – Wf  WN = 0	i.e., Ry = Wf + WN

Numerical Example:  Oil with S = .85 flows in pipe under pressure of 100 psi.  Pipe diameter is 3” and nozzle tip diameter is 1”

ρ=Sγg=1.65

D/d = 3

Q = π41122V2

= .716 ft3/s

V1 = 14.59 ft/s

V2 = 131.3 ft/s

Rx = 141.48 – 706.86 = 569 lbf

Rz = 10 lbf

This is force on nozzle

3.  Forces on Bends

Consider the flow through a bend in a pipe.  The flow is considered steady and uniform across the inlet and outlet sections.  Of primary concern is the force required to hold the bend in place, i.e., the reaction forces Rx and Ry which can be determined by application of the momentum equation.

Rx, Ry = reaction force on

bend i.e., force

required to hold

bend in place

Continuity:	0=ρV⋅A=−ρV1A1+ρV2A2

i.e., Q = constant = V1A1=V2A2

x-momentum:	Fx=ρuV⋅A			p1A1−p2A2cosθ+Rx=ρV1x−V1A1+ρV2xV2A2

= ρQV2x−V1x

y-momentum:  Fy=ρvV⋅A

p2A2sinθ+Ry−wf−wb=ρV1y−V1A1+ρV2yV2A2						   = ρQV2y−V1y

4. Problems involving Nonuniform Velocity Distribution

See text pp. 215 216

5. Force on a rectangular sluice gate

The force on the fluid due to the gate is calculated from the x-momentum equation:

Fx=ρuV⋅A

F1+FGW−Fvisc−F2=ρV1−V1A1+ρV2V2A2

usually can be neglected

FGW=F2−F1+ρQV2−V1+Fvisc

= γy22⋅y2b−γy12⋅y1b+ρQV2−V1

FGW=12bγy22−y12+ρQV2−V1

V1=Qy1bV2=Qy2b

ρQ2b1y2−1y1

Moment of Momentum Equation

See text pp. 221  229

Energy Equations

Derivation of the Energy Equation

The First Law of Thermodynamics

The difference between the heat added to a system and the work done by a system depends only on the initial and final states of the system; that is, depends only on the change in energy E: principle of conservation of energy

E = Q – W

E = change in energy

Q = heat added to the system

W = work done by the system

E = Eu + Ek + Ep = total energy of the system

potential energy

kinetic energy

Internal energy due to molecular motion

The differential form of the first law of thermodynamics expresses the rate of change of E with respect to time

dEdt=Q−W

rate of work being done by system

rate of heat transfer to system

Energy Equation for Fluid Flow

The energy equation for fluid flow is derived from Reynolds transport theorem with

Bsystem = E = total energy of the system (extensive property)

 = E/mass = e = energy per unit mass (intensive property)

=  + ek + ep

dEdt=ddtρed {V+ρeV⋅dA

This can be put in a more useable form by noting the following:

ek=Total KE of mass with velocity Vmass=ΔMV22ΔM=V22

V2=V

ep=EpΔM=γΔVzρΔV=gz		(for Ep due to gravity only)

rate of work			rate of change		flux of energy

done by system		of energy in CV		out of CV

(ie, across CS)

rate of heat

transfer to sysem

Rate of Work Components:  W=Ws+Wf

For convenience of analysis, work is divided into shaft work Ws and flow work Wf

Wf = net work done on the surroundings as a result of

normal and tangential stresses acting at the control

surfaces

= Wf pressure + Wf shear

Ws = any other work transferred to the surroundings

usually in the form of a shaft which either takes

energy out of the system (turbine) or puts energy into

the system (pump)

Flow work due to pressure forces  Wf p  (for system)

Note: here V uniform over A

System at time t + t

System at time t

CS

CV

Work = force  distance

at 2	W2 = p2A2  V2t

(on surroundings)

rate of work W2=p2A2V2=p2V2⋅A2

at 1	W1 = p1A1  V1t

neg. sign since pressure force on surrounding fluid acts in a direction opposite to the motion of the system boundary

W1=p1V1⋅A1

In general,

Wfp=pV⋅A

for more than one control surface and V not necessarily uniform over A:

Wfp=pV⋅dA=ρpρV⋅dA

Wf=Wfp+Wfshear

### Basic form of energy equation

h=enthalpy

#### Usually this term can be eliminated by proper choice of CV, i.e. CS normal to flow lines.  Also, at fixed boundaries the velocity is zero (no slip condition) and no shear stress flow work is done.  Not included or discussed in text!

Simplified Forms of the Energy Equation

Energy Equation for Steady One-Dimensional Pipe Flow

Consider flow through the pipe system as shown

Energy Equation (steady flow)

*Although the velocity varies across the flow sections the streamlines are assumed to be straight and parallel; consequently, there is no acceleration normal to the streamlines and the pressure is hydrostatically distributed, i.e., p/ +gz = constant.

*Furthermore, the internal energy u can be considered as constant across the flow sections, i.e. T = constant.  These quantities can then be taken outside the integral sign to yield

Recall that	Q=V⋅dA=VA

So that		ρV⋅dA=ρVA=m		mass flow rate

Define:		ρAV32dA=αρV3A2=αV22m

K.E. flux              K.E. flux for V=V=constant across pipe

i.e.,	      α=1AAVV3dA = kinetic energy correction factor

Nnote that:		 = 1 if V is constant across the flow section

 > 1 if V is nonuniform

laminar flow  = 2	       turbulent flow  = 1.05   1 may be used

Shaft Work

Shaft work is usually the result of a turbine or a pump in the flow system.  When a fluid passes through a turbine, the fluid is doing shaft work on the surroundings; on the other hand, a pump does work on the fluid

Ws=Wt−Wp		where Wt and Wp are

magnitudes of power	worktime

Using this result in the energy equation and deviding by g results in

mechanical part			    thermal part

Note: each term has dimensions of length

Define the following:

hp=Wpmg=WpρQg=WpγQ

ht=Wtmg

Head Loss

In a general fluid system a certain amount of mechanical energy is converted to thermal energy due to viscous action.  This effect results in an increase in the fluid internal energy.  Also, some heat will be generated through energy dissipation and be lost (i.e. -Q).  Therefore the term

from 2nd law

represents a loss in mechanical energy due to viscous stresses

Note that adding Q to system will not make hL = 0 since this also increases u.  It can be shown from 2nd law of thermodynamics that hL > 0.

Drop  over V and understand that V in energy equation refers to average velocity.

Using the above definitions in the energy equation results in (steady 1-D incompressible flow)

p1γ+α1V122g+z1+hp=p2γ+α2V222g+z2+ht+hL

form of energy equation used for this course!

Comparison of Energy Equation and Bernoulli Equation

Apply energy equation to a stream tube without any shaft work

Infinitesimal stream tube Þ 1=2=1

Energy eq :	p1γ+V122g+z1=p2γ+V222g+z2+hL

If hL = 0 (i.e.,  = 0) we get Bernoulli equation and conservation of mechanical energy along a streamline

Therefore, energy equation for steady 1-D pipe flow can be interpreted as a modified Bernoulli equation to include viscous effects (hL) and shaft work (hp or ht)

Summary of the Energy Equation

The energy equation is derived from RTT with

B = E = total energy of the system

 = e = E/M  = energy per unit mass

=  + 12V2+gz

internal	  KE	     PE

dEdt=ddtCVρed {V+CSρeV⋅dA=Q−W

heat addp1γ+z1+V122ged

work done

from 1st Law of Thermodynamics

##### Neglected in text presentation

W=Ws+Wp+Wv

shaft work done on or by system (pump or turbine)

pressure work done on CS

Viscous stress work on CS

Wp=CVpV⋅dA=CSρp/ρV⋅dA

Ws=Wt−Wp

Q−Wt+Wp=ddtCVρed {V+CSρe+p/eV⋅dA

For steady 1-D pipe flow (one inlet and one outlet):

- Streamlines are straight and parallel

 p/ +gz = constant across CS

- T = constant  u = constant across CS

3)	define	α=1ACSVV3dA = KE correction factor

	ρ2V3dA=αρV32A=αV22m

mechanical energy

Thermal energy

p1γ+α1V122g+z1+hp=p2γ+α2V222g+z2+ht+hL

# Note: each term has

units of length

V is average velocity (vector dropped) and

corrected by 

hp=Wp/mg

ht=Wt/mg

head loss

> 0  represents loss in mechanical energy due to viscosity

Application of the Energy, Momentum, and Continuity Equations in Combination

In general, when solving fluid mechanics problems, one should use all available equations in order to derive as much information as possible about the flow.  For example, consistent with the approximation of the energy equation we can also apply the momentum and continuity equations

Energy:

p1γ+α1V122g+z1+hp=p2γ+α2V222g+z2+ht+hL

Momentum:

one inlet and one outlet

 = constant

Fs=ρV22A2−ρV12A1=ρQV2−V1

Continuity:

A1V1 = A2V2 = Q = constant

Abrupt Expansion

Consider the flow from a small pipe to a larger pipe.  Would like to know hL = hL(V1,V2).  Analytic solution to exact problem is extremely difficult due to the occurrence of flow separations and turbulence.  However, if the assumption is made that the pressure in the separation region remains approximately constant and at the value at the point of separation, i.e, p1, an approximate solution for hL is possible:

Apply Energy Eq from 1-2 (1 = 2 = 1)

p1γ+z1+V122g=p2γ+z2+V222g+hL

Momentum eq. For CV shown (shear stress neglected)

Fs=p1A2−p2A2−Wsinα=ρuV⋅A

=ρV1(−V1A1)+ρV2(V2A2)

γA2LΔzL

=ρV22A2−ρV12A1

W  sin 

next divide momentum equation by A2

p1γ−p2γ−z1−z2=V22g−V12gA1A2=V12gA1A2A1A2−1

 A2

from energy equation		   using continuity



V222g−V122g+hL=V22g−V12gA1A2

hL=V222g+V122g1−2A1A2

continutity eq.

V1A1 = V2A2

A1A2=V2V1

hL=12gV22+V12−2V12A1A2

2V1V2

hL=12gV2−V12

Forces on Transitions

Example 7-6

Q = .707 m3/s

head loss = .1V222g

(empirical equation)

Fluid = water

p1 = 250 kPa

D1 = 30 cm

D2 = 20 cm

Fx = ?

First apply momentum theorem

Fx=ρuV⋅A

Fx + p1A1  p2A2 = V1(V1A1) + V2(V2A2)

Fx = Q(V2  V1)  p1A1 + p2A2

force required to hold transition in place

The only unknown in this equation is p2, which can be obtained from the energy equation.

p1γ+V122g=p2γ+V222g+hL		note: z1 = z2 and  = 1

p2=p1−γV222g−V122g+hL	drop in pressure

Fx=ρQV2−V1+A2p1−γV222g−V122g+hL−p1A1

p2

(note: if p2 = 0 same as nozzle)

In this equation,

continuity	A1V1 = A2V2

V2=A1A2V1

i.e.	V2 > V1

V1 = Q/A1 = 10 m/s

V2 = Q/A2 = 22.5 m/s

hL=.1V222g=2.58m

Fx = 8.15 kN		is negative x direction to hold

transition in place

Concept of Hydraulic and Energy Grade Lines

p1γ+α1V122g+z1+hp=p2γ+α2V222g+z2+ht+hL

Define 	HGL = pγ+z

point-by-point application is graphically displayed

EGL = pγ+z+αV22g

# HGL corresponds to pressure tap measurement + z

# EGL corresponds to stagnation tube measurement + z

EGL = HGL if V = 0

hL = fLDV22g

i.e., linear variation in L for D,

V, and f constant

EGL1 = EGL2 + hL

for hp = ht = 0

f = friction factor

f = f(Re)

pressure tap:  p2γ=h

h = height of fluid in

tap/tube

stagnation tube:  p2γ+αV222g=h

EGL1 + hp = EGL2 + ht + hL

EGL2 = EGL1 + hp  ht  hL

abrupt change due to hp or ht

fLDV22g

Helpful hints for drawing HGL and EGL

- EGL = HGL + V2/2g = HGL for V = 0

2.&3. hL=fLDV22g in pipe means EGL and HGL will slope

downward, except for abrupt changes due to ht or hp

p1γ+z1+V122g=p2γ+z2+V222g+hL

HGL2 = EGL1 - hL

hL=V22gfor abrupt expansion

4. p = 0  HGL = z

5. for hL=fLDV22g = constant  L

i.e., linearly increased for increasing L with slope fDV22g

EGL/HGL slope downward

6. for change in D  change in V

i.e.	V1A1 = V2A2

change in distance between HGL & EGL and slope

change due to change in hL

V1πD124=V2πD224



V1D12=V1D22

7.  If HGL < z then p/ < 0 	i.e., cavitation possible

condition for cavitation:

p=pva=2000Nm2

gage pressure	pva,g=pA−patm≈−patm=−100,000Nm2

pva,gγ≈−10m

9810 N/m3