Chapter 6 Momentum Analysis of Flow System

6.1 Momentum Equation

RTT with B = MV and  = V

FS+FB=ddtCVρVdV+CSVρVR⋅dA

V = velocity referenced to an inertial frame (non-accelerating)

VR = velocity referenced to control volume

FS = surface forces + reaction forces  (due to pressure and

viscous normal and shear stresses)

FB = body force (due to gravity)

6.2 Applications of the Momentum Equation

Initial Setup and Signs

- Jet deflected by a plate or a vane

- Flow through a nozzle

- Forces on bends

- Problems involving non-uniform velocity distribution

- Motion of a rocket

- Force on rectangular sluice gate

- Water hammer

6.3 Moment of Momentum Equation

6.1 Derivation of the Basic Equation

General form for moving but

non-accelerating reference frame

Recall RTT:	dBsysdt=ddtCVβρdV+CSβρVR⋅dA

VR=velocity relative to CS=V – VS=absolute – velocity CS

Subscript not shown in text but implied!

i.e., referenced to CV

small v in text and must be referenced to inertial reference frame

Let, 		B = MV = linear momentum

 = V

d(MV)dt=F=ddtCVVρdV+CSVρVR⋅dA

Must be relative to a non-accelerating inertial reference frame

Newton’s 2nd law

where	F = vector sum of all forces acting on the

control volume including both surface and

body forces

= FS + FB

FS = sum of all external surface forces acting at

the CS, i.e., pressure forces, forces

transmitted through solids, shear forces,etc.

FB = sum of all external

body forces, i.e.,

gravity force

Fx = p1A1 – p2A2 + Rx

Fy = -W + Ry

R = resultant force on fluid

in CV due to pw and w

free body diagram

i.e., reaction force on fluid

Important Features (to be remembered)

- Vector equation to get component in any direction must

use dot product

x equation

Fx=ddtCVρud {V+CSρuVR⋅dA

Carefully define coordinate system with forces positive in positive direction of coordinate axes

y equation

Fy=ddtCVρvd {V+CSρvVR⋅dA

z equation

Fz=ddtCVρwd {V+CSρwVR⋅dA

- Carefully define control volume and be sure to include all external body and surface faces acting on it.

For example,

(Rx,Ry) = reaction force on fluid

(Rx,Ry) = reaction force on nozzle

Velocity V must be referenced to a non-accelerating inertial reference frame.  Sometimes it is advantageous to use a moving (at constant velocity) reference frame.  Note VR = V – Vs is always relative to CS.

Text uses small v

i.e., in these cases V used for B also referenced to CV (i.e., V = VR)

- Steady vs. Unsteady Flow

Steady flow  ddtCVρVdV=0

- Uniform vs. Nonuniform Flow

CSVρVR⋅dA = change in flow of momentum across CS

= VVRA		uniform flow across A

- Fpres = pndA			V∇fd {V=Sfnds

f = constant, f = 0

= 0  for p = constant and for a closed surface

i.e., always use gage pressure

- Pressure condition at a jet exit

at an exit into the atmosphere jet pressure must be pa

6.2 Application of the Momentum Equation

1.  Jet deflected by a plate or vane

Consider a jet of water turned through a horizontal angle

CV and CS are for jet so that Fx and Fy are vane reactions forces on fluid

x-equation:	Fx=Fx=ddtρud {V+CSρuV⋅dA

Fx=CSρuV⋅A								steady flow

= ρV1x(−V1A1)+ρV2x(V2A2)

continuity equation:	A1V1 = A2V2 = Q

for A1 = A2

V1 = V2

Fx = Q(V2x – V1x)

y-equation:	Fy=Fy=CSρvV⋅A

Fy = V1y(– A1V1) + V2y(– A2V2)

= Q(V2y – V1y)

for above geometry only

where: 	V1x = V1    V2x = -V2cos   V2y = -V2sin  V1y = 0

note:		Fx and Fy are force on fluid

- Fx and -Fy are force on vane due to fluid

If the vane is moving with velocity Vv, then it is convenient to choose CV moving with the vane

i.e.,	 VR = V -  Vv  and V used for B also moving with vane

x-equation:	Fx=CSρuVR⋅dA

Fx = V1x[-(V – Vv)1A1] + V2x[-(V – Vv)2A2]

Continuity:	0 = ρVR⋅dA

i.e.,	(V-Vv)1A1 = (V-Vv)2A2 = (V-Vv)A

Qrel

Fx = (V-Vv)A[V2x – V1x]

Qrel

on fluid		V2x = (V – Vv)2x

For coordinate system moving with vane

V1x = (V – Vv)1x

Power = -FxVv			i.e., = 0 for Vv = 0

Fy = Qrel(V2y – V1y)

- Flow through a nozzle

Consider a nozzle at the end of a pipe (or hose).  What force is required to hold the nozzle in place?

### CV = nozzle

and fluid

 (Rx, Ry) = force required to hold nozzle in place

Assume either the pipe velocity or pressure is known.  Then, the unknown (velocity or pressure) and the exit velocity V2 can be obtained from combined use of the continuity and Bernoulli equations.

Bernoulli:	p1+γz1+12ρV12=p2+γz2+12ρV22      z1=z2

p1+12ρV12=12ρV22

Continuity:	A1V1 = A2V2 = Q

V2=A1A2V1=Dd2V1

p1+12ρV121−Dd4=0

Say p1 known:		V1=−2p1ρ1−Dd412

To obtain the reaction force Rx apply momentum equation in x-direction

Fx=ddtCVuρdV+CSρuV⋅dA

steady flow and uniform

flow over CS

=CSρuV⋅A

Rx + p1A1 – p2A2  	= V1(-V1A1) + V2(V2A2)

= Q(V2 - V1)

Rx	= Q(V2 - V1) - p1A1

To obtain the reaction force Ry apply momentum equation in y-direction

Fy=CSρvV⋅A=0	since no flow in y-direction

Ry – Wf  WN = 0	i.e., Ry = Wf + WN

Numerical Example:  Oil with S = .85 flows in pipe under pressure of 100 psi.  Pipe diameter is 3” and nozzle tip diameter is 1”

ρ=Sγg=1.65

D/d = 3

Q = π41122V2

= .716 ft3/s

V1 = 14.59 ft/s

V2 = 131.3 ft/s

Rx = 141.48 – 706.86 = 569 lbf

Rz = 10 lbf

This is force on nozzle

3.  Forces on Bends

Consider the flow through a bend in a pipe.  The flow is considered steady and uniform across the inlet and outlet sections.  Of primary concern is the force required to hold the bend in place, i.e., the reaction forces Rx and Ry which can be determined by application of the momentum equation.

Rx, Ry = reaction force on

bend i.e., force

required to hold

bend in place

Continuity:	0=ρV⋅A=−ρV1A1+ρV2A2

i.e., Q = constant = V1A1=V2A2

x-momentum:	Fx=ρuV⋅A			p1A1−p2A2cosθ+Rx=ρV1x−V1A1+ρV2xV2A2

= ρQV2x−V1x

y-momentum:  Fy=ρvV⋅A

p2A2sinθ+Ry−wf−wb=ρV1y−V1A1+ρV2yV2A2						   = ρQV2y−V1y

4. Problems involving Nonuniform Velocity Distribution

See text pp. 192  194

5. Motion of a Rocket

See text pp. 194  198

6. Force on a rectangular sluice gate

The force on the fluid due to the gate is calculated from the x-momentum equation:

Fx=ρuV⋅A

F1+FGW−Fvisc−F2=ρV1−V1A1+ρV2V2A2

usually can be neglected

FGW=F2−F1+ρQV2−V1+Fvisc

= γy22⋅y2b−γy12⋅y1b+ρQV2−V1

FGW=12bγy22−y12+ρQV2−V1

V1=Qy1bV2=Qy2b

ρQ2b1y2−1y1

- Water Hammer

See text pp. 198  204

6.3 Moment of Momentum Equation

See text pp. 248  259