# Chapter 8 Flow in Conduits

Entrance and developed flows

Le = f(D, V, , )

i theorem   Le/D = f(Re)

Laminar flow: Recrit  2000, i.e., for Re < Recrit laminar

Re > Recrit turbulent

Le/D = .06Re		from experiments

Lemax = .06RecritD  138D

maximum Le for laminar flow

Turbulent flow:

#### Re	   Le/D

4000	     18

104	     20

105	     30

106	     44

107	     65

108	     95

i.e., relatively shorter than for laminar flow

LeD  4.4Re16

from experiment

Laminar vs. Turbulent Flow

laminar			turbulent			spark photo

Reynolds 1883 showed difference depends on Re = VDν

Shear-Stress Distribution Across a Pipe Section

Continuity: Q1 = Q2 = constant, i.e., V1 = V2 since A1 = A2

Momentum:	Fs=ρuV⋅A

=ρV1−V1A1−ρV2V2A2

=ρQV2−V1=0

pA−p+dpdsdsA−ΔWsinα−τ2πrds=0

ΔW=γAds		sinα=dzds

−dpdsdsA−γAdsdzds−τ2πrds=0

 Ads

τ=r2−ddsp+γz

 varies linearly from 0.0 at r = 0 (centerline) to max (= w) at r = r0 (wall), which is valid for laminar and turbulent flow.

Laminar Flow in Pipes

### τ=μdVdy=−μdVdr=r2−ddsp+γz

y = wall coordinate = ro  r  dVdr=dVdydydr=−dVdy

dVdr=−r2μ−ddsp+γz

V=−r24μ−ddsp+γz+C

Vro=0⇒C=ro24μ−ddsp+γz

no slip condition

Exact solution to Navier-Stokes equations for laminar flow in circular pipe

where

Q=V⋅dA

=0roVr2πrdr

dA=rdrdθ=rdr2π

Q=πro48μ−ddsp+γz

For a horizontal pipe,

where  L = length of pipe = ds

Energy equation:

### p1γ+V122g+z1=p2γ+V222g+z2+hL

Define friction factor f=8τwρV2		Cf=τw12ρV2

friction coefficient for pipe flow

boundary layer flow

Darcy – Weisbach Equation, which is valid for both laminar and turbulent flow.

Friction factor definition based on turbulent flow analysis where thus n=6, m=3 and r=3 such that i=1,2,3 = f=8τwρV2, , k/D; or f=f(Re, k/D) where k=roughness height.   For turbulent flow f determined from turbulence modeling since exact solutions not known, as will be discussed next.

For laminar flow f not affected k and f(Re) determined from exact analytic solution to Navier-Stokes equations.

Exact solution:

For laminar flowthus n=4, m=3 and r=1 such that =constant.  The constant depends on duct shape (circular, rectangular, etc.) and is referred to as Poiseuille number=Po.  Po=4 for circular duct.

f=32μρroV=64μρVD=64Re

or	hf=hL=32μLVγD2		 hf = head loss due to friction

for z=0: as per Hagen!

Stability and Transition

Stability:  can a physical state withstand a disturbance and still return to its original state.

In fluid mechanics, there are two problems of particular interest: change in flow conditions resulting in (1) transition from one to another laminar flow; and (2) transition from laminar to turbulent flow.

(1)  Example of transition from one to another laminar flow: Centrifugal instability for Couette flow between two rotating cylinders when centrifugal force > viscous force , which is predicted by small-disturbance/linear stability theory.

(2)	Transition from laminar to turbulent flow

Not all laminar flows have different equilibrium states, but all laminar flows for sufficiently large Re become unstable and undergo transition to turbulence.

Transition:  change over space and time and Re range of laminar flow into a turbulent flow.

Recr=Uδυ ~ 1000, δ = transverse viscous thickness

Retrans > Recr 		with 	xtrans ~ 10-20 xcr

Small-disturbance/linear stability theory also predicts Recr with some success for parallel viscous flow such as plane Couette flow, plane or pipe Poiseuille flow, boundary layers without or with pressure gradient, and free shear flows (jets, wakes, and mixing layers).

No theory for transition, but recent Direct Numerical Simualtions is helpful.

In general: Retrans=Retrans(geometry, Re, pressure gradient/velocity profile shape, free stream turbulence, roughness, etc.)

Criterion for Laminar or Turbulent Flow in a Pipe

Recrit  2000		flow becomes unstable

Retrans  3000		flow becomes turbulent

Re = VD/

Turbulent Flow in Pipes

Continuity and momentum:

Energy:  	hf=Lγ−ddsp+γz

Combining:  define  = friction factor

##### hf=Lρg⋅2ro⋅18ρV2f

hf=f⋅LD⋅V22g	Darcy – Weisbach Equation

f = f(Re, k/D) = still must be determined!

Re=VDν			k = roughness

#### Description of Turbulent Flow

Most flows in engineering are turbulent:  flows over vehicles (airplane, ship, train, car), internal flows (heating and ventilation, turbo-machinery), and geophysical flows (atmosphere, ocean).

V (x, t) and p(x, t) are random functions of space and time, but statistically stationary flaws such as steady and forced or dominant frequency unsteady flows display coherent features and are amendable to statistical analysis, i.e. time and space (conditional) averaging.  RMS and other low-order statistical quantities can be modeled and used in conjunction with averaged equations for solving practical engineering problems.

Turbulent motions range in size from the width in the flow δ to much smaller scales, which come progressively smaller as the Re = Uδ/υ increases.

Physical description:

(1)  Randomness and fluctuations:

Turbulence is irregular, chaotic, and unpredictable.  However, for statistically stationary flows, such as steady flows, can be analyzed using Reynolds decomposition.

u=u+u'	   u=1Tt0t0+Tu dT	 u'=0   u'2=1Tt0t0+Tu'2 dT  etc.

u = mean motion

u'= superimposed random fluctuation

u'2= Reynolds stresses; RMS = u'2

Triple decomposition is used for forced or dominant frequency flows

u=u+u''+u'

Where u'' = organized component

(2)  Nonlinearity

Reynolds stresses and 3D vortex stretching are direct result of nonlinear nature of turbulence.  In fact, Reynolds stresses arise from nonlinear convection term after substitution of Reynolds decomposition into NS equations and time averaging.

(3)  Diffusion

Large scale mixing of fluid particles greatly enhances diffusion of momentum (and heat), i.e.,

Reynolds Stresses:  		−ρu'iu'j>> τij=μεijviscous stress

Isotropic eddy viscosity:

(4)  Vorticity/eddies/energy cascade

Turbulence is characterized by flow visualization as eddies, which vary in size from the largest Lδ (width of flow) to the smallest. The largest eddies have velocity scale U and time scale Lδ/U. The orders of magnitude of the smallest eddies (Kolmogorov scale or inner scale) are:

LK = Kolmogorov micro-scale =

LK =  O(mm) >> Lmean free path = 6 x 10-8 m

Velocity scale = (νε)1/4=  O(10-2m/s)

Time scale = (ν/ε)1/2=  O(10-2s)

Largest eddies contain most of energy, which break up into successively smaller eddies with energy transfer to yet smaller eddies until LK is reached and energy is dissipated at rate  by molecular viscosity.

Richardson (1922):

Lδ	Big whorls have little whorls

Which feed on their velocity;

And little whorls have lesser whorls,

LK	And so on to viscosity (in the molecular sense).

(5)  Dissipation

ℓ0=Lδu0=k k=u'2+v'2+w'2 =0 (U)Reδ=u0ℓ0υ=big

Energy comes from largest scales and fed by mean motion

ε = rate of dissipation = energy/time

Dissipation occurs at smallest scales

=u02τo		τo=ℓ0u0

=u03l0	independent υ	LK=υ3ε14

The mathematical complexity of turbulence entirely precludes any exact analysis.  A statistical theory is well developed; however, it is both beyond the scope of this course and not generally useful as a predictive tool.  Since the time of Reynolds (1883) turbulent flows have been analyzed by considering the mean (time averaged) motion and the influence of turbulence on it; that is, we separate the velocity and pressure fields into mean and fluctuating components.

It is generally assumed (following Reynolds) that the motion can be separated into a mean (u, v, w,p) and superimposed  turbulent fluctuating (u΄, v΄, w΄, p΄) components, where the mean values of the latter are 0.

u=u+u'				p=p+p'

v=v+v'				and for compressible flow

w=w+w'			ρ=ρ+ρ' and T=T+T'

where (for example)

and t1sufficiently large that the average is independent of time

u=1t1t0t0+t1udt

Thus by definition u'=0, etc.  Also, note the following rules which apply to two dependent variables f and g

f=f	f+g=f+g

f⋅g=f⋅g

∂f∂s=∂f∂s		fds=fds

f = (u, v, w, p)

s = (x, y, z, t)

The most important influence of turbulence on the mean motion is an increase in the fluid stress due to what are called the apparent stresses.  Also known as Reynolds stresses:

τij'=−ρui'uj'

−ρu	    −ρu'v'		−ρu'w'

Symmetric 2nd order tensor

=	−ρu'v'	    −ρv		−ρv'w'

−ρu'w'	    −ρv'w'	−ρw

The mean-flow equations for turbulent flow are derived by substituting V=V+V' into the Navier-Stokes equations and averaging.  The resulting equations, which are called the Reynolds-averaged Navier-Stokes (RANS) equations are:

Continuity	∇⋅V=0 i.e. ∇⋅V=0 and ∇⋅V'=0

Momentum	ρDVDt+ρ∂∂xjui'uj'=−ρgk−∇p+μ∇2V

or		ρDVDt=−ρgk−∇p+∇⋅τij

u1 = u	x1 = x

u2 = v	x2 = y

u3 = w	x3 = z

τij=μ∂ui∂xj+∂uj∂xi−ρui'uj'

τij'

Comments:

- equations are for the mean flow

- differ from laminar equations by Reynolds stress terms =

- influence of turbulence is to transport momentum from one point to another in a similar manner as viscosity

- since ui'uj' are unknown, the problem is indeterminate: the central problem of turbulent flow analysis is closure!

4 equations and 4 + 6 = 10 unknowns

#### Turbulence Modeling

Closure of the turbulent RANS equations require the determination of −ρu'v', etc.  Historically, two approaches were developed: (a) eddy viscosity theories in which the Reynolds stresses are modeled directly as a function of local geometry and flow conditions; and (b) mean-flow velocity profile correlations, which model the mean-flow profile itself.  The modern approaches, which are beyond the scope of this class, involve the solution for transport PDE’s for the Reynolds stresses, which are solved in conjunction with the momentum equations.

- eddy-viscosity: theories

−ρu'v'=μt∂u∂y

In analogy with the laminar viscous stress, i.e., t  mean-flow rate of strain

The problem is reduced to modeling t, i.e.,

t = t(x, flow at hand)

Various levels of sophistication presently exist in modeling t

μt=ρVtLt

turbulent length scale

where Vt and Lt are based on large scale turbulent motion

turbulent velocity scale

The total stress is

τtotal=μ+μt∂u∂y

molecular viscosity

eddy viscosity

(for high Re flow t >> )

Mixing-length theory (Prandtl, 1920)

based on kinetic theory of gases

−ρu'v'=cρu'2v'2

u'2=ℓ1∂u∂y

ℓ1 and ℓ2are mixing lengths which are analogous to molecular mean free path, but much larger

v'2=ℓ2∂u∂y



Known as a zero equation model since no additional PDE’s are solved, only an algebraic relation

distance across shear layer

ℓ=ℓy

= f(boundary layer, jet, wake, etc.)

Although mixing-length theory has provided a very useful tool for engineering analysis, it lacks generality.  Therefore, more general methods have been developed.

One and two equation models

C = constant

k = turbulent kinetic energy

=

 = turbulent dissipation rate

Governing PDE’s are derived for k and  which contain terms that require additional modeling.  Although more general than the zero-equation models, the k- model also has definite limitation; therefore, relatively recent work involves the solution of PDE’s for the Reynolds stresses themselves.  Difficulty is that these contain triple correlations that are very difficult to model.  Most recent work involves direct and large eddy simulation of turbulence.

- mean-flow velocity profile correlations

As an alternative to modeling the Reynolds stresses one can model mean flow profile directly for wall bounded flows such as pipes/channels and boundary layers.  For simple 2-D flows this approach is quite good and will be used in this course.  For complex and 3-D flows generally not successful.  Consider the shape of a turbulent velocity profile for wall bounded flow.

Note that very near the wall laminar must dominate since

−ρuiuj= 0 at the wall (y = 0) and in the outer part turbulent stress will dominate.  This leads to the three-layer concept:

Inner layer: 	viscous stress dominates

Outer layer:	turbulent stress dominates

Overlap layer:	both types of stress important

- laminar sub-layer (viscous shear dominates)

= f(, w, , y)		note: not f()

and =D and

y = ro  r

for pipe flow

From dimensional analysis

u+=fy+			law-of-the-wall

where:

u* = friction velocity = τwρ

y+=yuν

very near the wall:

  w  constant =        

i.e.,

u+=y+		0 < y+ < 5

- outer layer (turbulent shear dominates)

note: independent of  and actually also depends on dpdx

velocity defect law

From dimensional analysis

- overlap layer (viscous and turbulent shear important)

It is not that difficult to show that for both laws to overlap, f and g are logarithmic functions:

Inner region:

Outer region:

yuu2νdfdy+=yuuδdgdη; valid at large y+ and small η.

f(y+)

g(η)

Therefore, both sides must equal universal constant, κ−1

(inner variables)

(outer variables)

κ, A, and B are pure dimensionless constants

κ	=	0.41		Von Karman constant

Values vary somewhat depending on different exp. arrangements

B	=	5.5	       (or 5.0)

The difference is due to loss of intermittency in duct flow. A = 0 means small outer layer

A	=	2.35		BL flow

=	0.65		pipe flow

Note that the y+ scale is logarithmic and thus the inner law only extends over a very small portion of 

Inner law region < .2

And the log law encompasses most of the pipe/boundary-layer.  Thus as an approximation one can simply assume

is valid all across the shear layer.  This is the approach used in this course for turbulent flow analysis.  The approach is a good approximation for simple and 2-D flows (pipe and flat plate), but does not work for complex and 3-D flows.

# Velocity Distribution and Resistance in Smooth Pipes

Assume log-law is valid across entire pipe

friction

velocity

Vu=2.44lnrouν+1.34=ρVτo12=8f12

drop over bar:

12Ref812

1f=1.99logRe f12−1.02

constants adjusted using data

 1f=2logRe f12−.8		Re > 3000

Since f equation is implicit, it is not easy to see dependency on ρ, μ, V, and D

4000 < ReD < 105

Blasius (1911) power law curve fit to data

f(pipe)=0.316ReD−14

Drops weakly with pipe size

Near quadratic (as expected)

Nearly linear

Only slightly with μ

=0.241Lρ34μ14D−4.75Q1.75

laminar flow: Δp=8μLQπR4

Δp (turbulent) increases more sharply than Δp(laminar) for same Q; therefore, increase D for smaller Δp.  2D decreases Δp by 27 for same Q.

Combine with

Vumax=1+1.3f−1

Power law fit to velocity profile:

m = m(Re)

Viscous Distribution and Resistance – Rough Pipes

For laminar flow, effect of roughness is small; however, for turbulent flow the effect is large.  Both laminar sublayer and overlap layer are affected.

Inner layer:

= (y, k, , w)

not function of  as was case for smooth pipe (or wall)

u+ = u+(y/k)

Outer layer: unaffected

Overlap layer:

uR+=1κlnyk+constant				rough

uS+=1κlny++B					smooth

uS+−uR+=1κlnk++constant				k+=kuν

B(k+)

i.e., rough-wall velocity profile shifts downward by B(k+), which increases with k+.

Three regions of flow depending on k+

1.  k+ < 5	      hydraulically smooth (no effect of roughness)

2.  5 < k+ < 70		transitional roughness (Re dependence)

3.  k+ > 70		fully rough (independent Re)

For 3, 	ΔB=1κlnk+−3.5		from data

u+=1κlnyk+8.5≠fRe

fully rough flow

Vu=2.44lnDk+3.2

1f12=−2logkD3.7

###### Composite Log-Law

Smooth wall log law

u+=1κlny++B−ΔBk+

B*

B=5−1κln1+.3k+	from data

1f12=−2logkD3.7+2.51Re f12		Moody Diagram

=1.14−2logksD+9.35Re f12

There are basically three types of problems involved with uniform flow in a single pipe:

- Determine the head loss, given the kind and size of pipe along with the flow rate, Q = A*V

- Determine the flow rate, given the head, kind, and size of pipe

- Determine the pipe diameter, given the type of pipe, head, and flow rate

- Determine the head loss

The first problem of head loss is solved readily by obtaining f from the Moody diagram, using values of Re and ks/D computed from the given data.  The head loss hf is then computed from the Darcy-Weisbach equation.

f = f(ReD, ks/D)

= −Δpγ+z

ReD = ReD(V, D)

- Determine the flow rate

The second problem of flow rate is solved by trial, using a successive approximation procedure.  This is because both Re and f(Re) depend on the unknown velocity, V.  The solution is as follows:

  - solve for V using an assumed value for f and the Darcy-Weisbach equation

V=2ghfLD12⋅f−12

known from		note sign

given data

  - using V compute Re

  - obtain a new value for f = f(Re, ks/D) and reapeat as above until convergence

Or can use Re=f12=D32ν2ghfL12

scale on Moody Diagram

  - compute Ref12 and ks/D

  - read f

  - solve V from hf=fLDV22g

  - Q = VA

- Determine the size of the pipe

The third problem of pipe size is solved by trial, using a successive approximation procedure.  This is because hf, f, and Q all depend on the unknown diameter D.  The solution procedure is as follows:

  - solve for D using an assumed value for f and the Darcy-Weisbach equation along with the definition of Q

D=8LQ2π2ghf15⋅ f15

known from

given data

  - using D compute Re and ks/D

  - obtain a new value of f = f(Re, ks/D) and reapeat as above until convergence

Flows at Pipe Inlets and Losses From Fittings

For real pipe systems in addition to friction head loss these are additional so called minor losses due to

entrance and exit effects

can be large effect

- expansions and contractions

- bends, elbows, tees, and other fittings

- valves (open or partially closed)

For such complex geometries we must rely on experimental data to obtain a loss coefficient

K=hmV22g  		head loss due to minor losses

In general,

##### K = K(geometry, Re, /D)

dependence usually

not known

Loss coefficient data is supplied by manufacturers and also listed in handbooks.  The data are for turbulent flow conditions but seldom given in terms of Re.

Modified Energy Equation to Include Minor Losses (where ):

p1γ+z1+12gα1V12+hp=p2γ+z2+12gα2V22+ht+hf+hm									     hm=KV22g

Note:  hm does not include pipe friction and e.g. in elbows and tees,  this must be added to hf.

Flow in a bend:

−1rvθ2=−1ρ∂p∂r

centrifugal acceleration

i.e. ∂p∂r>0 which is an adverse pressure gradient in r direction.  The slower moving fluid near wall responds first and a swirling flow pattern results.

This swirling flow represents an

energy loss which must be added

to the hL.

Also, flow separation can result due to adverse longitudinal pressure gradients which will result in additional losses.

This shows potential flow is not a good approximate in internal flows (except possibly near entrance)

- Valves: enormous losses

- Entrances: depends on rounding of entrance

- Exit (to a large reservoir): K = 1

i.e., all velocity head is lost

- Contractions and Expansions

sudden or gradual

theory for expansion:

d

D

hL=V1−V222g

from continuity, momentum, and energy

(assuming p = p1 in separation pockets)

   KSE=1−d2D22=hmV122g

no theory for contraction:

KSC=.421−d2D2

from experiment

If the contraction or expansion is gradual the losses are quite different.  A gradual expansion is called a diffuser.  Diffusers are designed with the intent of raising the static pressure.

Cp=p2−p112ρV12

Cpideal=1−A1A22

Bernoulli and continuity equation

K=hmV22g=Cpideal−Cp	Energy equation

Actually very complex flow and

Cp = Cp (geometry, inlet flow conditions)

i.e., 	fully developed (long pipe) reduces Cp

thin boundary layer (short pipe) high Cp

(more uniform inlet profile)

Turbulent flow

K = .5

See textbook Table 8.2 for a table of the loss coefficients for pipe components