# Chapter 9 Flow over Immersed Bodies

Fluid flows are broadly categorized:

- Internal flows such as ducts/pipes, turbomachinery, open channel/river, which are bounded by walls or fluid interfaces: Chapter 8.

- External flows such as flow around vehicles and structures, which are characterized by unbounded or partially bounded domains and flow field decomposition into viscous and inviscid regions: Chapter 9.

  - Boundary layer flow: high Reynolds number flow around streamlines bodies without flow separation.

  - Bluff body flow: flow around bluff bodies with flow separation.

- Free Shear flows such as jets, wakes, and mixing layers, which are also characterized by absence of walls and development and spreading in an unbounded or partially bounded ambient domain: advanced topic, which also uses boundary layer theory.

Basic Considerations

Drag is decomposed into form and skin-friction contributions:

CDp			   Cf

<< 1	Cf > > CDp	  	streamlined body

1	CDp  > > Cf		bluff body

Streamlining: One way to reduce the drag

reduce the flow separationreduce the pressure drag

increase the surface area  increase the friction drag

Trade-off relationship between pressure drag and friction drag

Trade-off relationship between pressure drag and friction drag

Benefit of streamlining: reducing vibration and noise

Qualitative Description of the Boundary Layer

Flow-field regions for high Re flow about slender bodies:

w = shear stress

w  rate of strain (velocity gradient)

=

large near the surface where

fluid undergoes large changes to satisfy the no-slip condition

Boundary layer theory and equations are a simplified form of the complete NS equations and provides w as well as a means of estimating Cform.  Formally, boundary-layer theory represents the asymptotic form of the Navier-Stokes equations for high Re flow about slender bodies. The NS equations are 2nd order nonlinear PDE and their solutions represent a formidable challenge.  Thus, simplified forms have proven to be very useful.

Near the turn of the last century (1904), Prandtl put forth boundary-layer theory, which resolved D’Alembert’s paradox: for inviscid flow drag is zero.  The theory is restricted to unseparated flow.  The boundary-layer equations are singular at separation, and thus, provide no information at or beyond separation.  However, the requirements of the theory are met in many practical situations and the theory has many times over proven to be invaluable to modern engineering.

The assumptions of the theory are as follows:

Variable		   order of magnitude

u				U			O(1)

v				<<L		O()		 = /L

1/L			O(1)

1/			O(-1)

2			2

The theory assumes that viscous effects are confined to a thin layer close to the surface within which there is a dominant flow direction (x) such that u  U and v << u.  However, gradients across  are very large in order to satisfy the no slip condition; thus, >>.

Next, we apply the above order of magnitude estimates to the NS equations.

1  1       -1	        2    1        -2

elliptic

1  	     1		2    	-1

- 1

Retaining terms of O(1) only results in the celebrated boundary-layer equations

parabolic

Some important aspects of the boundary-layer equations:

- the y-momentum equation reduces to

i.e.,		p = pe = constant across the boundary layer

edge value, i.e., inviscid flow value!

from the Bernoulli equation:

constant

i.e.,

Thus, the boundary-layer equations are solved subject to a specified inviscid pressure distribution

- continuity equation is unaffected

- Although NS equations are fully elliptic, the boundary-layer equations are parabolic and can be solved using marching techniques

- Boundary conditions

u = v = 0		y = 0

u = Ue		y =

+ appropriate initial conditions @ xi

There are quite a few analytic solutions to the boundary-layer equations.  Also numerical techniques are available for arbitrary geometries, including both two- and three-dimensional flows.  Here, as an example, we consider the simple, but extremely important case of the boundary layer development over a flat plate.

Quantitative Relations for the Laminar Boundary Layer

Laminar boundary-layer over a flat plate:  Blasius solution (1908)		student of Prandtl

Note:  = 0 for a flat plate

u = v = 0	@ y = 0		u = U	@ y =

We now introduce a dimensionless transverse coordinate and a stream function, i.e.,

Substitution into the boundary-layer equations yields

Blasius Equation

@  = 0 	   @  = 1

The Blasius equation is a 3rd order ODE which can be solved by standard methods (Runge-Kutta).  Also, series solutions are possible.  Interestingly, although simple in appearance no analytic solution has yet been found.  Finally, it should be recognized that the Blasius solution is a similarity solution, i.e., the non-dimensional velocity profile f vs.  is independent of x.  That is, by suitably scaling all the velocity profiles have neatly collapsed onto a single curve.

Now, lets consider the characteristics of the Blasius solution:

vs. y

vs. y

value of y where u/U  = .99

i.e., 				see below

=

Wall shear stress:   or

Other:

displacement thickness

measure of displacement of inviscid flow due to boundary layer

momentum thickness

measure of loss of momentum due to boundary layer

H = shape parameter = =2.5916

Quantitative Relations for the Turbulent Boundary Layer

2-D Boundary-layer Form of RANS equations

requires modeling

Momentum Integral Analysis

Historically similarity and AFD methods used for idealized flows and momentum integral methods for practical applications, including pressure gradients.  Modern approach: CFD.

To obtain general momentum integral relation which is valid for both laminar and turbulent flow

For flat plate or  for general case

flat plate equation

momentum thickness

shape parameter

displacement thickness

Can also be derived by CV analysis as shown next for flat plate boundary layer.

#### Momentum Equation Applied to the Boundary Layer

Consider flow of a viscous fluid at high Re past a flat plate, i.e., flat palte fixed in a uniform stream of velocity.

Boundary-layer thickness arbitrarily defined by y = (where, is the value of y at u = 0.99U). Streamlines outside  will deflect an amount(the displacement thickness). Thus the streamlines move outward from at  to at .

Conservation of mass:

=0=

Assume incompressible flow (constant density):

Substituting defines displacement thickness:

is an important measure of effect of BL on external flow. Consider alternate derivation based on equivalent flow rate:

δ* Lam=/3

δ

δ* Turb=/8

Inviscid flow about δ* body

Flowrate between and of inviscid flow=actual flowrate, i.e., inviscid flow rate about displacement body = viscous flow rate about actual body

w/o BL - displacement effect=actual discharge

For 3D flow, in addition it must also be explicitly required that is a stream surface of the inviscid flow continued from outside of the BL.

Conservation of x-momentum:

= Fluid force on plate = - Plate force on CV (fluid)

Again assuming constant density and using continuity:

where,  is the momentum thickness (a function of x only), an important measure of the drag.

Special case 2D momentum integral equation for px = 0

Per unit span

Simple velocity profile approximations:

u(0) = 0              no slip

u(δ) = U             matching with outer flow

uy(δ)=0

Use velocity profile to get Cf() and () and then integrate momentum integral equation to get (Rex)

δ* = δ/3

θ = 2δ/15

H= δ*/θ= 5/2

10% error, cf. Blasius

Approximate solution Turbulent Boundary-Layer

Ret = 5×105 3×106	for a flat plate boundary layer

Recrit  100,000

as was done for the approximate laminar flat plate boundary-layer analysis, solve by expressing cf = cf () and  = () and integrate, i.e.

assume log-law valid across entire turbulent boundary-layer

neglect laminar sub layer and velocity defect region

at y = , u = U

cf ()

or

power-law fit

Next, evaluate

can use log-law or more simply a power law fit

# Note: can not be used to obtain  cf () since w

i.e., much faster growth rate than laminar boundary layer

or

almost linear

If the boundary layer is “tripped” by some roughness or leading edge disturbance, the boundary layer is turbulent from the beginning:

()

shear stress:

Alternate forms depending on experimental information and power-law fit used, etc.  (i.e., dependent on Re range.)

Total shear-stress coefficient

Some additional relations for larger Re are as follows:

Re > 107

Local

shear-stress coefficient

Finally, a composite formula that takes into account both the initial laminar boundary-layer (with translation at

ReCR = 500,000) and subsequent turbulent boundary layer

is 		105 < Re < 107

Bluff Body Drag

Drag of 2-D Bodies

First consider a flat plate both parallel and normal to the flow

=  		laminar flow

=  		turbulent flow

flow pattern

## vortex wake

typical of bluff body flow

# where Cp based on experimental data

=

= 2    using numerical integration of experimental data

Cf  = 0

For bluff body flow experimental data used for CD.

In general, Drag = f(V, L, , , c, t, , T, etc.)

from dimensional analysis

c/L

scale factor

Potential Flow Solution:

surface pressure

Flow Separation

Flow separation:

The fluid stream detaches itself from the surface of the body at
    sufficiently high velocities. Only appeared in viscous flow!!

Flow separation forms the region called ‘separated region’

Inside the separation region:

low-pressure, existence of recirculating/backflows

viscous and rotational effects are the most significant!

Important physics related to flow separation:

’Stall’ for airplane (Recall the movie you saw at CFD-PreLab2!)

Vortex shedding

(Recall your work at CFD-Lab2, AOA=16°! What did you see in your velocity-vector plot at the trailing edge of the air foil?)

Terminal Velocity

Z

Terminal velocity is the maximum velocity attained by a falling body when the drag reaches a magnitude such that the sum of all external forces on the body is zero. Consider a sphere using Newton’ Second law:

when terminal velocity is attained :

or

For the sphere

and

The terminal velocity is:

Magnus effect: Lift generation by spinning

Breaking the symmetry causes the lift!

Effect of the rate of rotation on the lift and drag coefficients of a smooth sphere:

Lift acting on the airfoil

Lift force: the component of the net force (viscous+pressure) that is perpendicular to the flow direction

Variation of the lift-to-drag ratio with angle of attack:

The minimum flight velocity:

Total weight W of the aircraft be equal to the lift

Effect of Compressibility on Drag: CD = CD(Re, Ma)

speed of sound = rate at which infinitesimal

disturbances are propagated from their source into undisturbed medium

< 0.3 flow is incompressible,

i.e.,   constant

Ma < 1		subsonic

Ma  1		transonic  (=1 sonic flow)

Ma > 1		supersonic

Ma >> 1		hypersonic

CD increases for Ma  1 due to shock waves and wave drag

# Macritical(sphere)  .6

Macritical(slender bodies)  1

For U > a:  	upstream flow is not warned of approaching

disturbance which results in the formation of shock waves across which flow properties and streamlines change discontinuously